Ausschreibungstext

Artikelnummer

208135

Typ

AXIS M7016 VIDEO ENCODER

Kurzbeschreibung

Video Netzwerk Server, 16 Eingänge, H.264, MJPEG, 720x576, microSDXC-Slots

Hauptmerkmale

Weitere Merkmale (Spezifikationen)

| Produktkennzeichnungen |  |
| --- | --- |
| EAN/GTIN | 7331021004115 |
| Hersteller | AXIS |

Weitere Produktinformationen

Zubehör

| AXIS 4IN1 SECURITY SCREWDRIVER | Schraubendreher Set, 4-in-1, Biteinsätze pasend für Axis Produkte, inkl. Tasche |
| --- | --- |
| AXIS ACC MAINS ADAP PS-P T-C | Netzteil, 12VDC, 2A, Hohlstecker |
| AXIS ACS 1 CORE DEVICE LICENSE | Axis Camera Station Software, Version 5.x Core Device, 1 Kanal, Papierlizenz! |
| AXIS ACS 1 UNIVERSAL DEVICE LICENSE | Axis Camera Station Software, Version 5.x Universal Device, 1 Kanal, Papierlizenz! |
| AXIS ACS 10 UNIVERSAL DEVICE LICENSE | Axis Camera Station Software, Version 5.x Universal Device, 10 Kanal, Papierlizenz! |
| AXIS ACS 16 CORE DEVICE LICENSE | Axis Camera Station Software, Version 5.x Core Device, 16 Kanal, Papierlizenz! |
| AXIS ACS 20 UNIVERSAL DEVICE LICENSE | Axis Camera Station Software, Version 5.x Universal Device, 20 Kanal, Papierlizenz! |
| AXIS ACS 32 CORE DEVICE LICENSE | Axis Camera Station Software, Version 5.x Core Device, 32 Kanal, Papierlizenz! |
| AXIS ACS 4 CORE DEVICE LICENSE | Axis Camera Station Software, Version 5.x Core Device, 4 Kanal, Papierlizenz! |
| AXIS ACS 4 UNIVERSAL DEVICE LICENSE | Axis Camera Station Software, Version 5.x Universal Device, 4 Kanal, Papierlizenz! |
| AXIS ACS CORE DEVICE E-LICENSE | Axis Camera Station Software, Core Device, E-Lizenz 1 Kanal, Sammelcode bei > 1 Stück |
| AXIS ACS UNIVERSAL DEVICE E-LICENSE | Axis Camera Station Software, Universal Device, E-Lizenz 1 Kanal, Sammelcode bei > 1 Stück |
| AXIS Companion VMS Version 4 | Video Management Software, für 16 Axis Kameras, kostenlos |
| AXIS LONG RANGE POE EXTENDER KIT | PoE Extender, PoE, PoE+, Innen, RJ45, bis 1000m Reichweite |
| AXIS RJ45 FIELD CONNECTOR 10 PCS | Netzwerkstecker, RJ45, male, CAT5/6/6a (AWG 22-26), MPTL- und PoE geeignet, 10 Stück |
| AXIS SURVEILLANCE CARD 128 GB | Speicherkarte, microSDXC, 128GB, Class 10, 50-80 MB/s, SD-Adapter, Axis zertifiziert |
| AXIS SURVEILLANCE CARD 128 GB 10P | Speicherkarte, microSDXC, 128GB, Class 10, 50-80 MB/s, Axis zertifiziert, 10 Stück |
| AXIS SURVEILLANCE CARD 256GB | Speicherkarte, microSDXC, 256GB, Class 10, V30, inkl. SD-Adapter |
| AXIS SURVEILLANCE CARD 256GB 10PCS | Speicherkarte, microSDXC, 256GB, Class 10, V30, inkl. SD-Adapter, 10 Stück |
| AXIS SURVEILLANCE CARD 64 GB | Speicherkarte, microSDXC, 64GB, Class 10, 20 MB/s, inkl. SD-Adapter, Axis zertifiziert |
| AXIS T8129 POE EXTENDER | PoE Extender, PoE, PoE+ kompatibel, RJ45, 100m Reichweite |
| AXIS T8310 CONTROL BOARD | Kontrollcenter, modular, mit Keypad, Joystick, Jog Dial |
| AXIS T8604 MEDIA CONVERTER SWITCH | Ethernet Medienkonverter, 2x 10/100Mbps RJ45, 2x 100/1000Mbps SFP |
| AXIS T8640 POE+ OVER COAX KIT | Medienkonverter, RJ45, BNC, Ethernet über Koax, PoE+, Base-/Device Set (2 Stück) |
| AXIS T8645 POE+ COAX COMPACT KIT | PoE+ über Coax Set, 1x AXIS T8641 Base Einheit, 1x AXIS T8643 Device Einheit |
| AXIS T8646 POE+ OVER COAX KIT | PoE+ OVER COAX Set, 6-Kanal: 1x T8646 POE+ OVER COAX BLADE, 6x T8642 POE+ OVER COAX DEVICE |
| AXIS T8648 POE+ COAX BLADE COM | PoE+ über Coax Set, 1x AXIS T8646 Einschubkarte, 6x AXIS T8643 Device Einheiten |

Empfehlungen

| jetrics network RJ45S-Cat6-small-FC | RJ45 Stecker Cat.6 AWG22-26, Feldkonfektionierbar, schmale und kurze Ausführung |
| --- | --- |
| Western Digital SD card,WDD256G1P0C,256GB,Class10 | MicroSDXC Speicherkarte, 256GB, Class 10, UHS-I U1, 128TBW, für Videoüberwachung |
| Western Digital WDD032G1P0C | MicroSDXC Speicherkarte, 32GB, Class 10, UHS-I U1, 16TBW, für Videoüberwachung |
| Western Digital WDD064G1P0C | MicroSDXC Speicherkarte, 64GB, Class 10, UHS-I U1, 32TBW, für Videoüberwachung |
| Western Digital WDD128G1P0C | MicroSDXC Speicherkarte, 128GB, Class 10, UHS-I U1, 64TBW, für Videoüberwachung |

Dienstleistungen

| AXIS M7016 EXT.WARRANTY |  |
| --- | --- |