Ausschreibungstext

Artikelnummer

220269

Typ

AXIS Q1659 BAREBONE

Kurzbeschreibung

Netzwerkkamera, Tag/Nacht, ohne Objektiv, 5472x3648, SFP, EF/EF-S Mount, PoE, 8-28VDC

Hauptmerkmale

Weitere Merkmale (Spezifikationen)

| Produktkennzeichnungen |  |
| --- | --- |
| EAN/GTIN | 7331021065055 |
| Hersteller | AXIS |

Weitere Produktinformationen

Zubehör

| AXIS 30 W MIDSPAN AC/DC 24 V | PoE+ Midspan, IEEE802.3at, IEEE802.3af, 1 Port, Innen, 30W, 24 V AC, 10-28 V DC |
| --- | --- |
| AXIS 30W MIDSPAN | PoE+ Midspan, IEEE802.3at, Type 2, Class 4, 1 Port, Innen, 100-240VAC |
| AXIS 30W OUTDOOR MIDSPAN | PoE Midspan, 1 Port, IEEE802.3at, 30W, IP66/67, -40°C - +65°C |
| AXIS 4IN1 SECURITY SCREWDRIVER | Schraubendreher Set, 4-in-1, Biteinsätze pasend für Axis Produkte, inkl. Tasche |
| AXIS ACS 1 CORE DEVICE LICENSE | Axis Camera Station Software, Version 5.x Core Device, 1 Kanal, Papierlizenz! |
| AXIS ACS 1 UNIVERSAL DEVICE LICENSE | Axis Camera Station Software, Version 5.x Universal Device, 1 Kanal, Papierlizenz! |
| AXIS ACS 10 UNIVERSAL DEVICE LICENSE | Axis Camera Station Software, Version 5.x Universal Device, 10 Kanal, Papierlizenz! |
| AXIS ACS 16 CORE DEVICE LICENSE | Axis Camera Station Software, Version 5.x Core Device, 16 Kanal, Papierlizenz! |
| AXIS ACS 20 UNIVERSAL DEVICE LICENSE | Axis Camera Station Software, Version 5.x Universal Device, 20 Kanal, Papierlizenz! |
| AXIS ACS 32 CORE DEVICE LICENSE | Axis Camera Station Software, Version 5.x Core Device, 32 Kanal, Papierlizenz! |
| AXIS ACS 4 CORE DEVICE LICENSE | Axis Camera Station Software, Version 5.x Core Device, 4 Kanal, Papierlizenz! |
| AXIS ACS 4 TO UNIVERSAL 1 UPG.LIC | Axis Camera Station, Upgrade Version 4.x auf 5.x Universal Device, 1 Kanal, Papierlizenz! |
| AXIS ACS 4 TO UNIVERSAL 10 UPG.LIC | Axis Camera Station, Upgrade Version 4.x auf 5.x Universal Device, 10 Kanal, Papierlizenz! |
| AXIS ACS 4 TO UNIVERSAL 20 UPG.LIC | Axis Camera Station, Upgrade Version 4.x auf 5.x Universal Device, 20 Kanal, Papierlizenz! |
| AXIS ACS 4 TO UNIVERSAL 5 UPG.LIC | Axis Camera Station, Upgrade Version 4.x auf 5.x Universal Device, 5 Kanal, Papierlizenz! |
| AXIS ACS 4 TO UNIVERSAL UPG E-LICENSE | Axis Camera Station, Upgrade Version 4.x nach 5.x Universal Device, E-Lizenz, 1 Kanal |
| AXIS ACS 4 UNIVERSAL DEVICE LICENSE | Axis Camera Station Software, Version 5.x Universal Device, 4 Kanal, Papierlizenz! |
| AXIS ACS CORE DEVICE E-LICENSE | Axis Camera Station Software, Core Device, E-Lizenz 1 Kanal, Sammelcode bei > 1 Stück |
| AXIS ACS CORE TO UNIVERSAL 1 UPG.LIC | Axis Camera Station, Upgrade Version 5.x Core auf Universal Device, 1 Kanal, Papierlizenz! |
| AXIS ACS CORE TO UNIVERSAL 10 UPG.LIC | Axis Camera Station, Upgrade Version 5.x Core auf Universal Device, 10 Kanal, Papierlizenz! |
| AXIS ACS CORE TO UNIVERSAL 20 UPG.LIC | Axis Camera Station, Upgrade Version 5.x Core auf Universal Device, 20 Kanal, Papierlizenz! |
| AXIS ACS CORE TO UNIVERSAL 5 UPG.LIC | Axis Camera Station, Upgrade Version 5.x Core auf Universal Device, 5 Kanal, Papierlizenz! |
| AXIS ACS CORE TO UNIVERSAL UPG E-LICENSE | Axis Camera Station, Upgrade Version 5.x Core to Universal Device, E-Lizenz, 1 Kanal |
| AXIS ACS UNIVERSAL DEVICE E-LICENSE | Axis Camera Station Software, Universal Device, E-Lizenz 1 Kanal, Sammelcode bei > 1 Stück |
| AXIS CONNECTOR A 2P2.5 STR 10PCS | Axis Anschlussblock, 10-polig, Stecker, Abstand: 2.50 mm, für I/O Ports, 10 Stück |
| AXIS CONNECTOR A 2P3.81 STR 10PCS | Axis Anschlussblock, 2-polig, Stecker, Abstand: 3.81 mm, für Stromversorgung, 10 Stück |
| AXIS CONNECTOR A 4P2.5 STR 10PCS | Axis Anschlussblock, 4-polig, Stecker, Abstand: 2.50 mm, für I/O Ports, 10 Stück |
| AXIS I/O INDICATION LED 4P | Rotlicht LED, zur Montage an Axis Kameras, 4 Stück, Zubehörteil |
| AXIS LONG RANGE POE EXTENDER KIT | PoE Extender, PoE, PoE+, Innen, RJ45, bis 1000m Reichweite |
| AXIS M12(F)-RJ45(M) CABLE 0.5M (1.6FT) | Kabel, CAT5e, M12 (Female, D-Code) auf RJ-45 (Male), 0,5m |
| AXIS NETWORK CABLE COUPLER INDOOR | RJ45 Koppler, Buchse-Buchse, nur für Innenraum |
| AXIS NETWORK CABLE COUPLER INDOOR SLIM | RJ45 Koppler, Buchse-Buchse, nur für Innenraum, kompakte Ausführung |
| AXIS NETWORK CABLE COUPLER IP66 | RJ45 Koppler, Buchse-Buchse, für Außen, IP66 |
| AXIS POWER SUPPLY PS24 240 W | Versorgungsnetzteil, 24VDC, 10A, 240W, 100-240 VAC, IP67, -40° - +70°C |
| AXIS RJ45 FIELD CONNECTOR 10 PCS | Netzwerkstecker, RJ45, male, CAT5/6/6a (AWG 22-26), MPTL- und PoE geeignet, 10 Stück |
| AXIS SURVEILLANCE CARD 128 GB | Speicherkarte, microSDXC, 128GB, Class 10, 50-80 MB/s, SD-Adapter, Axis zertifiziert |
| AXIS SURVEILLANCE CARD 128 GB 10P | Speicherkarte, microSDXC, 128GB, Class 10, 50-80 MB/s, Axis zertifiziert, 10 Stück |
| AXIS SURVEILLANCE CARD 256GB | Speicherkarte, microSDXC, 256GB, Class 10, V30, inkl. SD-Adapter |
| AXIS SURVEILLANCE CARD 256GB 10PCS | Speicherkarte, microSDXC, 256GB, Class 10, V30, inkl. SD-Adapter, 10 Stück |
| AXIS SURVEILLANCE CARD 64 GB | Speicherkarte, microSDXC, 64GB, Class 10, 20 MB/s, inkl. SD-Adapter, Axis zertifiziert |
| AXIS T8129 POE EXTENDER | PoE Extender, PoE, PoE+ kompatibel, RJ45, 100m Reichweite |
| AXIS T8134 60W MIDSPAN | High PoE Midspan, 1 Port, 60W, Gigabit, -10 - +45°C, 100-240V IEEE802.3at, IEEE802.3af |
| AXIS T8144 60W INDUSTRIAL MIDSPAN | High PoE Midspan, 1 Port, 60W, Gigabit, -40 - +75°C, 20-60VDC DIN Montage |
| AXIS T8351 MK II MICROPHONE 3.5MM | Mikrofon, omnidirektional, analog, 3,5mm Klinkenstecker, diskrete Ausführung, IP65 |
| AXIS T8353A MICROPHONE 3.5MM | Mikrofon, omnidirektional, 3,5mm Klinkenstecker, diskrete Ausführung |
| AXIS T8415 WIRELESS INSTALLATION TOOL | Installations Tool, zur Axis Kamera Einrichtung, WLAN, Akku, USB, PoE+, ONVIF |
| AXIS T8516 POE+ NETWORK SWITCH | Gigabit Switch, managed, 16 PoE+ Ports, 2x SFP/RJ45 Uplinks, 240W PoE, 19", 1HE |
| AXIS T8604 MEDIA CONVERTER SWITCH | Ethernet Medienkonverter, 2x 10/100Mbps RJ45, 2x 100/1000Mbps SFP |
| AXIS T8606 MEDIA CONV SWITCH 24VDC | Ethernet Medienkonverter, 2x 10/100Mbps RJ45, 2x 100/1000Mbps SFP, 24VDC |
| AXIS T8611 SFP MODULE LC.LX | SFP Modul, Gigabit, 1310nm, Single mode, 10km, LC Anschluss |
| AXIS T8612 SFP MODULE LC.SX | SFP Modul, Gigabit, 850nm, Multi mode, 550m, LC Anschluss |
| AXIS T8613 SFP MODULE 1000BASE-T | SFP Modul, Gigabit, 1000BASE-T, RJ45 |
| AXIS T8640 POE+ OVER COAX KIT | Medienkonverter, RJ45, BNC, Ethernet über Koax, PoE+, Base-/Device Set (2 Stück) |
| AXIS T8645 POE+ COAX COMPACT KIT | PoE+ über Coax Set, 1x AXIS T8641 Base Einheit, 1x AXIS T8643 Device Einheit |
| AXIS T90D20 IR-LED | LED Infrarot Scheinwerfer, 850nm, 10-120°, austauschbare Linsen, 12-24V, 18W, IP66, IK9 |
| AXIS T90D20 POE IR-LED | LED Infrarot Scheinwerfer, 850nm, 10-120°, austauschbare Linsen, PoE, 20W, IP66, IK09 |
| AXIS T90D25 POE W-LED | LED Weißlicht Scheinwerfer, 6000K, 10-120°, austauschbare Linsen, PoE, 25W, IP66, IK09 |
| AXIS T90D25 W-LED | LED Weißlicht Scheinwerfer, 6000K, 10-120°, austauschbare Linsen, 12-24V, 24W, IP66, IK9 |
| AXIS T90D30 IR-LED | LED Infrarot Scheinwerfer, 850nm, 10-120°, austauschbare Linsen, 12-24V, 46W, IP66, IK9 |
| AXIS T90D30 POE IR-LED | LED Infrarot Scheinwerfer, 850nm, 10-120°, austauschbare Linsen, PoE, 49W, IP66, IK09 |
| AXIS T90D35 POE W-LED | LED Weißlicht Scheinwerfer, 6000K, 10-120°, austauschbare Linsen, PoE, 47W, IP66, IK09 |
| AXIS T90D35 W-LED | LED Weißlicht Scheinwerfer, 6000K, 10-120°, austauschbare Linsen, 12-24V, 42W, IP66, IK9 |
| AXIS T90D40 IR-LED | LED Infrarot Scheinwerfer, 850nm, 10-120°, austauschbare Linsen, 24V, 100W, IP66, IK09 |
| AXIS T93C10 OUTDOOR HOUSING | Wetterschutzgehäuse, für Axis Q16 und P13 Kameras, Aktive Kühlung, Heizung, IK10, IP66 |
| AXIS TU1001-V MICROPHONE | Mikrofon, analog, für Innenmontage, 3,5 mm Klinke, IK10 |
| AXIS TU1001-VE MICROPHONE | Mikrofon, analog, für Außenmontage, 3,5 mm Klinke, IK10, IP66 |

Empfehlungen

| Jetrics jetcon-RJ45S-Cat6-small-FC | RJ45 Stecker Cat.6 AWG22-26, Feldkonfektionierbar, schmale und kurze Ausführung |
| --- | --- |
| Western Digital SD card,WDD256G1P0C,256GB,Class10 | MicroSDXC Speicherkarte, 256GB, Class 10, UHS-I U1, 128TBW, für Videoüberwachung |
| Western Digital WDD032G1P0C | MicroSDXC Speicherkarte, 32GB, Class 10, UHS-I U1, 16TBW, für Videoüberwachung |
| Western Digital WDD064G1P0C | MicroSDXC Speicherkarte, 64GB, Class 10, UHS-I U1, 32TBW, für Videoüberwachung |
| Western Digital WDD128G1P0C | MicroSDXC Speicherkarte, 128GB, Class 10, UHS-I U1, 64TBW, für Videoüberwachung |

Dienstleistungen

| CON-BER-M | 2D Planung Medium mittlere Projektkomplexität ca. 25 Kameras |
| --- | --- |
| VIDEOR CON-KOMP | Kompatibilitätsprüfung für Kameras, NVR, VMS-Systeme (ein Gerätepaar) |
| VIDEOR CUS-WSG-EINBAU-B | Montage Kamera / Objektiv in ein Wetterschutzgehäuse |
| VIDEOR Vorkonfiguration IP-Kamera | IP Konfiguration, Funktionsprüfung, FW-Update für 1x IP-Kamera |