Specifications text

Article number

220931

Type

IED-62M2812MCA

Short description

1/2,8" Network Dome, Fix, 1920x1080,Day&Night, WDR, 2,8-12mm, EPOC, IR, IP66

Main Features

1/2,8" Sony STARVIS CMOS

Resolution max. 1920x1080, 25fps frame rate

Motorised vary focal lens 2,8~12mm with P-Iris

EPOC Ethernet and Power over Coax transmission

Wide Dynamic Range, T-WDR (120dB)

Intelligent self learning video analysis (IntelliVision Technology Inside)

Quad Streaming: H.264, H.265 x3, MJPEG x1, Smart Codec

Micro SD Card (SDHC, SDXC)

1 x Alarm in and output as well as audio support

Infrared illumination up to 30m

Easy installation, IP66, PoE IEEE 802.3af

Support ONVIF Profile S and Profile T

Other Features (Specifications)

| Observe (62 px/m) | 16 - 55 m |
| --- | --- |
| Recognize (125 px/m) | 8 - 26 m |
| Identify (250 px/m) | 4 - 13 m |
| Detect (25px/m) | 40 - 130 m |
| Manufacturer order number | 220931 |
| Brand | eneo |

Additional Productinformation

Related products

| AK-8B | Camera junction box, Wall and Ceiling mounting |
| --- | --- |
| AK-9B | Camera junction box for DH-6, Dome-WB4 |
| eneo Site Manager | Management Software for eneo IP products |
| IAM-4MU1001M0A | EPOC Signal Converter, BNC to 2-wire cable transmission for EPOC Cameras, Transceiver |
| IAM-6MC1001M0A | Ethernet over Coax Transceiver Data/Power supply 270Mbps, 57VDC power input |
| IAM-6MC1001MRA | Ethernet Coax Receiver Data & Power over Coax 1 x RJ-45, 1 x BNC |
| IED-FMH1 | Ceiling mount adapter with white cover for fix dome cameras, SN-series |

Recommendations

| LACKIERUNG Fix-Dome-Kamera | Extra charge for special paint fixed dome in RAL color |
| --- | --- |

Services

| LACKIERUNG Fix-Dome-Kamera | Extra charge for special paint fixed dome in RAL color |
| --- | --- |
| LACKIERUNG Grundkosten | Painting basic costs once per RAL color and delivery lot |
| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |