Specifications text

Article number

217917

Type

eneo VM-FHD43M

Short description

43" (109cm) LCD Monitor FHD, 1920x1080, LED, HDMI, VGA Composite, Metal cabinet

Main Features

Professional 43" (109 cm) LCD monitor

Industrial design built to run 24x7x365 continuously

Maximum resolution 1920x1080 pixels

LED Back Light

Fast response time 5ms

Perfect presentation through digital 3D filtering and noise reduction

Different video inputs HDMI, VGA, Composite

Built-in speakers

Supply voltage: 240-100VAC, 50/60Hz

Metal cabinet including stand

Other Features (Specifications)

| General |  |
| --- | --- |
| Housing material | Metal |
| Display |  |
| Viewing Angle Horizontal | 178 ° |
| Viewing Angle Vertical | 178 ° |
| Screen diagonal (cm) | 109 cm |
| Screen diagonal (inches) | 43 " |
| Display | Anti-glare display, Color TFT-LCD, Hard coating (3H) |
| Brightness (typical) | 350 cd/m² |
| Backlight | LED |
| Contrast ratio (typical) | 3500:1 |
| Response time (typical) | 5 ms |
| Refresh rate | 60 Hz |
| Functions |  |
| Menu languages | English |
| Interfaces / Inputs/ Outputs |  |
| Audio Inputs | 1 |
| Transmission Technology | HDCP 1.4, HDCP 2.2, HDMI 1.4a |
| Control Interfaces | no |
| Video Inputs | 2x Composite (FBAS), HDMI, VGA |
| Parts supplied |  |
| Scope of delivery | manual,power cord,IR remote controller,VGA cable,feet set |
| Product labeling: |  |
| Manufacturer order number | 217917 |
| Series | eneo Monitor |
| Brand | eneo |

Additional Productinformation

Related products

Recommendations

Services