Specifications text

Article number

225718

Type

IED-68M3611MAA 9005

Short description

1/2.5" Network Dome, Fixed, 3840x2160, Day&Night, WDR, 3,6-11mm, IR, IP66, Black

Main Features

1/2,5" Sony STARVIS CMOS

4K UHD Resolution (3840x2160), 25fps frame rate

Motorised varifocal lens 3,6~11mm with P-Iris

Intelligent self-learning video analysis (IntelliVision inside)

Wide Dynamic Range, True WDR (120dB)

Quad Streaming: H.264, H.265 x3, MJPEG x1, Smart Codec ROI Hi-Stream

Micro SD Card (SDHC, SDXC)

1 x Alarm in and output as well as audio support

Infrared illumination up to 30m

PoE IEEE 802.3af Class 0, 12VDC, IP66

Supports ONVIF Profile S and Profile T

Colour RAL 9005 black

Other Features (Specifications)

| Observe (62 px/m) | 40 - 95 m |
| --- | --- |
| Recognize (125 px/m) | 17 - 47 m |
| Identify (250 px/m) | 9 - 23 m |
| Detect (25px/m) | 94 - 230 m |
| Manufacturer order number | 225718 |
| Brand | eneo |

Additional Productinformation

Related products

| AK-8B RAL 9005 | Camera junction box for wall and ceiling mount, black |
| --- | --- |

Recommendations

| LACKIERUNG Fix-Dome-Kamera | Extra charge for special paint fixed dome in RAL color |
| --- | --- |

Services

| LACKIERUNG Fix-Dome-Kamera | Extra charge for special paint fixed dome in RAL color |
| --- | --- |
| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |