Specifications text

Article number

225158

Type

eneo IED-62M2812MAA 7016

Short description

1/2.8" Network Dome, Fixed, 1920x1080, Day&Night, WDR, 2,8-12mm, IR, Anthracite Gray

Main Features

1/2,8" Sony STARVIS CMOS

Resolution max. 1920x1080, 25fps frame rate

Motorised vary focal lens 2.7~12mm with P-Iris

Intelligent self-learning video analysis (IntelliVision inside)

Wide Dynamic Range, T-WDR (120dB)

Quad Streaming: H.264, H.265 x3, MJPEG x1, Smart Codec ROI Hi-Stream

Micro SD Card (SDHC, SDXC)

1 x Alarm in and output as well as audio support

Infrared illumination up to 30m

PoE IEEE 802.3af Class 0,12VDC, IP66

Supports ONVIF Profile S and Profile T

Colour RAL 7016 anthracite gray

Other Features (Specifications)

| General |  |
| --- | --- |
| Heating | No |
| Vandalism-resistant | Yes |
| Lighting | Infrared |
| Dimensions (WxHxD) (extended) | 99x144mm |
| Weight | 0.776 kg |
| SDK | upon request |
| Text overlay | available |
| Color (Housing) | RAL7016 |
| Color (dome) | clear |
| Housing type | Outside |
| Housing material | Aluminium |
| Power consumption | 8.5 W |
| Mounting type | Ceiling, Wall |
| ONVIF | ONVIF Profile S, ONVIF Profile T |
| Power over Ethernet | IEEE 802.3af |
| Protection | IP66 |
| Power supply | 12VDC, Poe |
| Temperature Range (Operating) | -20°C ~ +50°C |
| Wide Dynamic Range (WDR) | WDR 120dB |
| Certifications | IK10 |
| Camera |  |
| Backlight compensation | Yes |
| Low Speed Shutter | Yes |
| Resolution standard | 2 MP |
| Recording sensor | CMOS |
| Chip size | 1/2,8" |
| Flickerless function | Yes |
| Camera shape | Minsters |
| Day/night mode | Day,Night,Automatic |
| White balance | Automatic,Manual |
| Photosensitivity | 0,03 Lux bei F1,6,40 IRE |
| System | True Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| Display |  |
| Image resolution max. | 1920x1080 |
| DORI |  |
| Watch (62px/m) | 13,8 - 52,6 m |
| Detect (125px/m) | 6,5 - 26 m |
| Identify (250px/m) | 3,1 - 13 m |
| Perceiving (25px/m) | 34,6 - 130 m |
| Functions |  |
| Analysis | Loitering Detection, Manipulation (TD), Motion Detection |
| Audio support | Yes |
| Motion detector | Yes |
| Digital Noise Reduction (DNR) | Yes |
| Multicast | Yes |
| Turning range | 359 ° |
| Alarm processing | recording internal storage (SD card),E-Mail,FTP,Boost,HTTP,TCP,UDP,HTTPS,XML,alarm out,Alarm Forwarding (Preset Call of eneo PTZ Dome) |
| Audio | 2 Way Audio |
| Browser | Edge: MJPEG setup limited, Google Chrome: MJPEG setup limited, Mozilla: MJPEG setup limited, Safari: MJPEG setup limited |
| Menu languages | Chinese, English, French, German, Japanese, Korean, Russian |
| Password | Administrator and User |
| Vertical tilt range | 0 ~ +90° |
| Internal memory | microSDHC card, microSDXC card |
| Notes |  |
| Remark | Note that electronic adjustment aid such as Zoom / Focus or Pan / Tilt functions are not designed for continuous operation.Permanent use may affect the life of the camera,Colour matching accessories on request,To guarantee you an optimum user experience, |
| Interfaces / Inputs/ Outputs |  |
| Alarm inputs | 1 |
| External connections | Alarm output,Alarm input,Audio input/output,RJ-45,12VDC input (DC plug),connection cable approx.40cm |
| Control Interfaces | Ethernet |
| Parts supplied |  |
| Scope of delivery | manual,bag of screws |
| Lens |  |
| Horizontal angle of view | 100 - 33° |
| Vertical angle of view | 56 - 18° |
| Iris Control | P-Iris |
| Focal length | 2,8 - 12 mm |
| MOD (Minimum Object Distance) | 0.5 m |
| Lens Type | Varifocal (motorized) |
| Zoomfactor (optical) | 4 |
| Video |  |
| Image transfer rate max. | 30 fps |
| Resolution (extended) | 1920x1080 (16:9),1440x1080 (4:3),1280x1024 (5:4),1024x768 (4:3),1280x720 (16:9),800x600 (4:3),800x480 (16:9),704x576 (4:3),704x480 (4:3),640x480 (4:3),400x240 (4:3),352x288 (4:3), |
| Data stream types | Quad Streaming |
| Video Compression | H.264, H.264+, H.265, H.265+, M-JPEG |
| Product labeling: |  |
| Manufacturer order number | 225158 |
| Series | eneo SN-Serie |
| Brand | eneo |

Additional Productinformation

Related products

Recommendations

Services