Specifications text

Article number

221889

Type

IAM-5SE1004MUP

Short description

Gigabit Switch, Unmanaged 60W, 4x PoE, Industrial Design -40°C~+75°C, PD port

Main Features

Gigabit PoE Switch, DIN Rail

4x 10/100/1000 PoE Ports, IEEE 802.3ab/af/at/u

1 x SFP Gigabit uplink port

1 x RJ45 Gigabit uplink port, PD-capable (60W PoE input)

PoE watchdog (reboot automatically)

Surge protection IEC61000-4-5 (6KV)

Temperature range -40° C ~ +75° C

Other Features (Specifications)

| Number of PoE ports | 4 |
| --- | --- |
| Design | DIN Rail Module |
| Dimensions (WxHxD) (extended) | 46x110x163mm |
| Remark | We recommend the use edp. number: 216568 PoE Injector NE-60PoE to ensure an interference-free function with power supply via PD port |
| Application environment | inside |
| ESD protection | IEC61000-4-2 |
| Weight | 540 kg |
| Scope of delivery | manual,power cord,Mounting Bracket,PSU |
| Multicast | No |
| Eneo _netzwerkstandards | IEEE 802.3 10BaseT, IEEE 802.3u 100BaseTX, IEEE 802.3x Flow Control |
| Internal Switching Capacity (Gbit) | 12 Gbps |
| Internal Switching Capacity (MP per second) | 8.9 Mpps |
| Ethernet interface | 1000Base-T, 100Base-TX, 10Base-T |
| Type | Unmanaged |
| Power consumption | 60 W |
| Fan | No |
| Management | No |
| Mounting type | DIN Rail |
| Manufacturer order number | 221889 |
| PoE Budget (Total) | 60 W |
| Ports | 1x 10/100/1000Base-T (PoE-PD), 1x 1000Base-X SFP, 4x 10/100/1000Base-T |
| Power over Ethernet | IEEE 802.3af, IEEE 802.3at |
| Rack installation | No |
| Series | eneo Switch |
| Power supply | 48-56VDC, Poe |
| Temperature Range (Operating) | -40°C ~ +75°C |
| VLAN | Yes |
| Brand | eneo |

Additional Productinformation

Related products

Recommendations

Services