Specifications text

Article number

221890

Type

IAR-7SM1024MMB

Short description

Gigabit Switch, Managed, 24xGb SFP slots, 8xGb RJ45 ports, 4x10Gb uplink SFP slots

Main Features

Managed L3 switch

24x Gigabit SFP slots, 8 shared RJ45 ports

4x SFP Gigabit uplink ports

Fast ring redundancy, reconfiguration

Provision of a web-based layer 3 network management

Support high speed data forwarding

Very suitable for forwarding large amounts of video data in security monitoring

ESD protection IEC61000-4-5 standard

Supports two high performance power supplies for redundant operation

Fast installation and commissioning

Temperature range -10 ° C ~ + 50 ° C

Suitable for wall / rack mounting and desktop use

Other Features (Specifications)

| Manufacturer order number | 221890 |
| --- | --- |
| Brand | eneo |

Additional Productinformation

Related products

| eneo Site Manager | Management Software for eneo IP products |
| --- | --- |
| NE-150W12 | Redundant power module for IAR-7SM1024MMB, input 100-240VAC, output 12VDC,150W |
| RJT-1000SFP | RJ45- SFP Transceiver Modul, 1000Base-T, 0°~ +70°C |
| SFP-0850M10 | SFP Modul, 1.25Gbps, 550m, Multimode, 850nm, Duplex LC Connector, 0°~+70°C |
| SFP-0850M10-10G | SFP Modul, 10Gbps, 300m, Multimode, 850nm, Duplex LC Connector, -5°~+70°C |
| SFP-0850M40 | SFP Modul, 1,25Gbps, 550m, Multimode, 850nm, Duplex LC Connector, -40° ~ +70°C |
| SFP-1310M40 | SFP Modul, 1.25Gbps, 550m, Multimode, 1310nm, Duplex LC Connector, -40°~+75°C |
| SFP-1310S10 | SFP Modul, 1.25Gbps, 20km, Singlemode, 1310nm, Duplex LC Connector, 0°~+70°C |
| SFP-1310S10-10G | SFP Modul, 10Gbps, 10km, Singlemode, 1310nm, Duplex LC Connector, -5°~+70°C |
| SFP-1310S40 | SFP Modul, 1.25Gbps, 20km, Singlemode, 1310nm, Duplex LC Connector -40°~+75°C |

Recommendations

Services