Specifications text

Article number

220196

Type

ISD-59P0010P0A

Short description

1/1,7" Network Dome, fixed, Day/Night, 4000x3000, H.265, 1,29mm, 360°, infrared, IP66

Main Features

1/1.7" Sony STARVIS CMOS-Sensor

1.29mm fixed lens FishEye camera

360° Panorama-, Corridor-, Quad-, Panomorph-View, ceiling or wall

Integration in eneo Center Software

True Day & Night

Resolution max. 4000x3000@20fps

Quad Stream (H.264/H.265x3, MJPEGx1)

Cross Web-Browsing

True-WDR (Digital Wide Dynamic Range)

Framerate increase on motion detection (Smart Codec)

IR-LED Illumination integrated

Compact design for indoor application

Other Features (Specifications)

| Alarm inputs | 1 |
| --- | --- |
| Audio support | Yes |
| Resolution standard | 8 MP |
| Recording sensor | CMOS |
| Lighting | Infrared |
| Motion detector | No |
| Image resolution max. | 4000x3000 |
| Image transfer rate max. | 30 fps |
| Horizontal angle of view | 360° |
| Focal length | 1,29 mm |
| Chip size | 1/1,7" |
| Digital Noise Reduction (DNR) | Yes |
| Dimensions (WxHxD) (extended) | 58,7x151,4mm |
| Alarm processing | recording internal storage (SD card),E-Mail,FTP,Boost,HTTP,TCP,UDP,HTTPS,XML,alarm out |
| Audio | 2 Way Audio |
| Resolution (extended) | 4000x3000 (4:3),3000x3000,2560x2560,2160x2160,2048x2048,1280x1280,640x640,480x480 |
| Remark | To guarantee you an optimum user experience,we recommend that you only use components of the same brand in a system. If several brands are used in combination,eneo cannot guarantee functionality. If combined use is necessary,we offer a compatibility check |
| Watch (62px/m) | 25 m |
| Browser | Edge: MJPEG setup limited, Google Chrome: MJPEG setup limited, Mozilla: MJPEG setup limited, Safari: MJPEG setup limited |
| Data stream types | Quad Streaming |
| Detect (125px/m) | 5 m |
| External connections | Alarm output,Alarm input,Audio input/output,RJ-45,12VDC input (DC plug),connection cable approx.40cm |
| Identify (250px/m) | 5 m |
| Camera shape | Minsters |
| Scope of delivery | manual,bag of screws |
| Menu languages | Chinese, English, French, German, Japanese, Korean, Russian |
| Multicast | Yes |
| Sunshade canopy | No |
| Day/night mode | Automatic,Day,Night |
| Text overlay | available |
| Perceiving (25px/m) | 60 m |
| Integrated wall mount | No |
| White balance | Automatic,fluorescent,Manual,Outdoor,incandescent |
| Backlight compensation | No |
| Housing material | Metal |
| Heating | Yes |
| Internal memory | microSD card, microSDXC card |
| Camera Type | Fisheye Camera |
| Photosensitivity | 0,3 Lux bei F2,8 |
| Low Speed Shutter | No |
| MOD (Minimum Object Distance) | 0.1 m |
| Mounting type | Ceiling, Wall |
| Lens Type | Prime lens |
| ONVIF | ONVIF Profile S, ONVIF Profile T |
| Manufacturer order number | 220196 |
| Power over Ethernet | IEEE 802.3af |
| Protection | IP66 |
| Series | eneo SN-Serie |
| Control Interfaces | Ethernet |
| Power supply | 12VDC, Poe |
| System | True Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| Temperature Range (Operating) | -20°C ~ +50°C |
| Vandalism-resistant | Yes |
| Video Compression | H.264, H.264+, H.265, H.265+, M-JPEG |
| Wide Dynamic Range (WDR) | WDR 120dB |
| Wiper | No |
| Certifications | IK10 |
| Brand | eneo |

Additional Productinformation

Related products

Recommendations

Services