Specifications text

Article number

225289

Type

IAM-5SG1008MMB

Short description

Gigabit Switch, Managed, 240W, 8x PoE, 4x SFP, 1000Mbps, Industrial Design, -40°C~+75°C

Main Features

Gigabit PoE Switch, DIN Rail, fan-less

4x SFP Gigabit Uplink Ports

8x 10/100/1000 PoE Ports, IEEE 802.3af/at

Supports VLAN, RTSP, "Fast Ring", and QoS

Up to 30W per port, Total ≤240W (optional power supply)

Fast Ring-Redundant, Reconfiguration

Supports redundant power supply connection

ESD protection IEC61000-4-5 Standard

Web configuration, Web management

Fault Relay, Console Port, Status LED

PoE reset button for each port

Temperature range -40°C~+75°C

Other Features (Specifications)

| Number of PoE ports | 8 |
| --- | --- |
| Design | DIN Rail Module |
| Dimensions (WxHxD) (extended) | 46,5x110x163mm |
| Remark | External power supply specification: 100~240VAC,150W,30x70x150mm,-30°~+70°,To achieve the specified PoE budget (total) value,an appropriate optional power supply unit must be used. |
| ESD protection | IEC61000-4-2, IEC61000-4-5 |
| Weight | 750 kg |
| Scope of delivery | manual,power cord,Mounting Bracket,PSU |
| Multicast | Yes |
| Eneo _netzwerkstandards | IEEE 802.1D Spanning Tree Protocol, IEEE 802.3 10BaseT, IEEE 802.3u 100BaseTX, IEEE 802.3x Flow Control |
| Security | Fast Ring, Password, Port Isolation, Port Speed Setting, Radius Authentication |
| Internal Switching Capacity (Gbit) | 24 Gbps |
| Internal Switching Capacity (MP per second) | 17.9 Mpps |
| Ethernet interface | 1000Base-T, 100Base-TX, 10Base-T |
| Type | Layer2 Switch |
| Fan | No |
| Management | Yes |
| Mounting type | DIN Rail, Wall |
| Manufacturer order number | 225289 |
| PoE Budget (Total) | 240 W |
| Ports | 4x 1000Mbps SFP, 8x 10/100/1000Base-T |
| Power over Ethernet | IEEE 802.3af, IEEE 802.3at |
| Rack installation | No |
| Series | eneo Switch |
| Power supply | 48-57 VDC |
| Temperature Range (Operating) | -40°C ~ +75°C |
| VLAN | Yes |
| Brand | eneo |

Additional Productinformation

Related products

Recommendations

Services