Specifications text

Article number

225247

Type

MED-55F0036M0A

Short description

1/2,7" HD Dome, Fix, Day&Night, 2592x1944, 3.6mm, Infrared, 12VDC, IP66

Main Features

1/2,7" CMOS Image Sensor

Resolution max. 2592x1920

Multisignal: HD-TVI, AHD, CVI, CVBS

Fixed 3.6mm day & night lens

Removable infrared cut filter

2D/-3D-DNR noise reduction

Digital Wide Dynamic Range (D-WDR), Defog

LDC function(Lens Distortion Correction)

OSD menu control and signal switching possible via joystick in the cable feed

Compact housing for outdoor IP66

Other Features (Specifications)

| Dimensions (HxWxD) | 85x90mm |
| --- | --- |
| Serial interfaces protocols | TVI-UTC |
| Series | eneo COAXIZE, EX-Series |
| Resolution Pixels | 2560x1440, 2592x1944, 960x576 |
| Resolution Pixels | 2560x1440, 2592x1944, 960x576 |
| System | True Day&Night |
| Sensor size | 1/2.7" |
| Imager | CMOS |
| Day&Night switch | ICR |
| Sensitivity | 0,05 Lux at F2,0, 50 IRE, at F1,6 |
| Low Speed Shutter | no |
| Digital Noise Reduction | yes |
| Backlight Compensation | no |
| Wide Dynamic Range (WDR) | D-WDR |
| Motion Detector | no |
| Lens Type | fixed focal length |
| Focal Length | 3,6 mm |
| Digital Zoom | no |
| Observe (62 px/m) | 29 m |
| Recognize (125 px/m) | 14 m |
| Identify (250 px/m) | 6 m |
| Detect (25px/m) | 70 m |
| Horizontal Angle of View | 87,4° |
| Vertical Angle of View | 47° |
| MOD (Mininum Object Distance) | 0,2 m |
| llumination | Infrared |
| Housing | Outdoor |
| Color (housing) | RAL9016 |
| Color (bubble) | clear |
| Hosuing material | Aluminium |
| Heater | no |
| Blower | no |
| Video outputs | AHD, Composite (CVBS), CVI, HD-TVI |
| Alarm inputs | nein |
| Alarm outputs | no |
| Serial interfaces | Coaxitron (HD Protocol) |
| Mounting | Ceiling mount, Wall mount |
| Supply voltage | 12VDC |
| Power consumption | 3,7W |
| Vandal proof | no |
| Operating temperature | -20°C ~ +50°C |
| Protection rating | IP66 |
| Signal format | AHD @ 5MP, Composite (FBAS), CVI @ 4MP, HD-TVI @ 5MP |
| Camera type | Dome |
| Frame rate | 20fps @ 2592x1944p, 25/30fps @ 2560x1440p |
| Aperture range (F) | F2.0 |
| Illumination range | approx. 20m |
| Weight | 360g |
| Manufacturer order number | 225247 |
| Brand | eneo |

Additional Productinformation

<h2>4MP HD Analog Kamera für die Outdoor-Überwachung</h2><p>Ein robustes Aluminiumgehäuse, eine maximale Auflösung von satten 4 Megapixeln (2592x1944) und vier frei wählbare Videosignalformate (HD-TVI, AHD, CVI, CVBS) zeichnen diese eneo 4MP Kamera aus. Mit ihrem IP66-geschützten Gehäuse und einem Betriebstemperaturbereich von -20°C bis 50°C widersteht die 4MP HD Analog Kamera vom Typ MED-55F0036M0A dauerhaft auch unwirtlichen Wetterverhältnissen. Dank dieser Eigenschaften kann die eneo 4MP Kamera im Fix-Dome Design ohne Probleme in der Outdoor-Videoüberwachung eingesetzt werden. <br><br></p><h3>Videosicherheit rund um die Uhr</h3><p>Das Tag-Nacht-Objektiv der eneo 4MP HD Analog Kamera verfügt über eine Festbrennweite von 3,6mm. Für die automatische Umstellung auf Infrarotbetrieb ist MED-55F0036M0A mit einem schwenkbaren IR-Sperrfilter und 18 Infrarot-LEDs ausgestattet. Die Beleuchtungsreichweite beträgt rund 20m. In Tests wurden folgende DORI-Distanzen für die eneo 4MP Kamera ermittelt: Wahrnehmen 70m, Beobachten 29m, Erkennen 14m und Identifizieren 6m. Damit eignet sich der eneo Fix-Dome mit Multisignalunterstützung unter anderem sehr gut für die Überwachung von Tankstellen-Shops oder den Eingangsbereich von Supermärkten. <br><br></p><h3>Funktionen für stetig hohe Bildqualität</h3><p>Tageszeiten- und wetterbedingte Veränderungen der Licht- und Sichtverhältnisse stellen jede Kamera vor Herausforderungen. In der professionellen Videoüberwachung, wo besonders hohe Anforderungen an die Bildqualität gelten, verfügen leistungsstarke Modelle wie die 4MP HD Analog Kamera von eneo über eine Reihe von Bildoptimierungsfunktionen, die für eine konstant hohe Videoqualität sorgen. So verfügt MED-55F0036M0A über eine digitale Rauschunterdrückung (2D-/3DNR), die im Zusammenspiel mit D-WDR detailreiche, gleichmäßige und aussagekräftige Bilder gewährleistet. Etwaige Bildverzeichnung durch Objektivverzerrungen gleicht die eneo 4MP Kamera automatisch mittels der LDC-Funktion aus, während Bildbeeinträchtigungen durch Smog, Nebel oder Niesel mittels Defog-Funktion behoben werden.</p>

Related products

| AK-8B | Camera junction box, Wall and Ceiling mounting |
| --- | --- |
| AK-9B | Camera junction box for DH-6, Dome-WB4 |
| MAM-5DC1004M0A | Distributor, 1x BNC Input HD-TVI, CVI, AHD, CVBS, 4x BNC Output |
| MAM-5MM2203M0A | Converter, 4K HD-TVI, HD-CVI, AHD, auf HDMI, Indoor 12VDC |
| PS12DC-11 | Plug-in Power Supply Unit, 100-240VAC/12VDC (1,0A), regulated, with DC Plug |
| TVA-1201TRA | UTP Transmitter+Receiver, UTP, BNC, 250m, Indoor, HD TVI CVI, AHD, CVBS, Active, 12V |
| TVA-1201TRP | UTP Transmitter+Receiver, UTP, BNC, 150m, Indoor, HD TVI CVI, AHD, CVBS, Passive |

Recommendations

Services

| LACKIERUNG Fix-Dome-Kamera | Extra charge for special paint fixed dome in RAL color |
| --- | --- |
| LACKIERUNG Grundkosten | Painting basic costs once per RAL color and delivery lot |
| Vorkonfiguration analoge Kamera | Preconfiguration 1x analog camera Function test, FW update |