Specifications text

Article number

227036

Type

IER-28R160405A

Short description

Network Video Recorder, 16x IP, H.265, HDMI/VGA, 16x PoE, 4TB HDD

Main Features

Other Features (Specifications)

| Product labeling: |  |
| --- | --- |
| Brand | eneo |

Additional Productinformation

Related products

| eneo Center | Remote Software IER-25xx, IER-28xx, IER-38xx, MSR-24xx, MSR-25xx, MER-Serie |
| --- | --- |
| eneo Center mobile | Mobile App for IER-25xx, IER- IER-28xx. IER-38xx, IEM-38xx MSR-24xx, MSR-25xx |
| eneo HDD-2000SATA Purple | Hard Disk Drive 2TB, SATA, for eneo Video Recorders, expansion |
| eneo HDD-3000SATA Purple | Hard Disk Drive 3TB, SATA, for eneo Video Recorders, expansion |
| eneo HDD-4000SATA Purple | Hard Disk Drive 4TB, SATA, for eneo Video Recorders, expansion |
| eneo HDD-6000SATA Purple | Hard Disk Drive 6TB, SATA, for eneo Video Recorders, expansion |
| eneo HDD-8000SATA Purple | Hard Disk Drive 8TB, SATA, for eneo Video Recorders, expansion |
| eneo KBD-2USB | System Keyboard with 3-Axis Joystick, Jog Shuttle, USB, RS-485, 12VDC, |
| IEM-38R640005A | 64-Channel Virtual Matrix for IER-, MSR-Series, H.264, H.265 640Mbps, w/o HDD, HDMI |

Recommendations

Services

| CON-KOMP | Compatibility check for cameras, NVR, VMS systems (one pair of devices) |
| --- | --- |