Specifications text

Article number

226625

Type

eneo MPP-62A0020M0B

Short description

1/2,8" HD Analogue Dome, PTZ, Day&Night, 20x, 4,7-94mm, 1920x1080, WDR, IP66

Main Features

Attention successor article no. 243176

1/2,8" Sony STARVIS CMOS-Sensor

20x Auto Focus Zoom, 4.7 ~ 94mm / 16x Digital

Resolution max. 1920x1080 (Full HD)

Separate outputs for HD signals and composite

Signal formats: HD-TVI, AHD selectable, composite

Real time WDR (Wide Dynamic Range) 25 / 30fps

Compact design for in- and outdoor applications

3D DNR noise reduction, auto-calibration

Protected areas, Defog function

Alarm inputs and outputs, 12VDC/24VAC, IK10, IP66

Other Features (Specifications)

| General |  |
| --- | --- |
| Fan | Yes |
| Vandalism-resistant | Yes |
| Lighting | No |
| Dimensions (WxHxD) (extended) | 150x154mm |
| Weight | 1.6 kg |
| Text overlay | available |
| Color (Housing) | white |
| Color (dome) | clear |
| Housing type | Inside, Outside |
| Housing material | Aluminium |
| Power consumption | 12 W |
| Mounting type | Ceiling, Wall |
| Protection | IP66 |
| Power supply | 12VDC, 24VAC |
| Temperature Range (Operating) | -20°C ~ +50°C |
| Wide Dynamic Range (WDR) | WDR 120dB |
| Certifications | IK10 |
| Camera |  |
| Backlight compensation | Yes |
| Low Speed Shutter | Yes |
| Recording sensor | CMOS |
| Chip size | 1/2,8" |
| Flickerless function | Yes |
| Camera shape | PTZ |
| Signal-to-noise ratio | 50 dB |
| Day/night mode | Automatic,Day,Night |
| White balance | Automatic,Manual,Light bulb,Fluorescent light,Outdoor |
| Photosensitivity | 0,35 Lux bei F1,6,50 IRE |
| System | True Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| Display |  |
| Image resolution max. | 1280x720,720x576,1920x1080 |
| DORI |  |
| Watch (62px/m) | 28 - 550 m |
| Detect (125px/m) | 13 - 270 m |
| Identify (250px/m) | 7 - 130 m |
| Perceiving (25px/m) | 70 - 1370 m |
| Functions |  |
| Motion detector | Yes |
| Digital Noise Reduction (DNR) | Yes |
| Turning range | 360 ° |
| Home Position | Selectable, Tour, Preset, Pattern |
| Menu languages | English, French, German, Italian, Polish, Portuguese, Spanish |
| Tours | 8 selectable tours |
| Speed (manual control) | 380 °/sec |
| Speed (Target Approach) | 380 ° |
| Interfaces / Inputs/ Outputs |  |
| Alarm inputs | 4 |
| Number of alarm outputs | 2 |
| External connections | Alarm in/output,voltage input,RS-485,Composite (FBAS),HD-TVI / AHD BNC,connection cable approx.25cm |
| Interface Protocols | Fastrax, Pelco D, Pelco P, UTC (HD Signal Format) |
| Control Interfaces | Coaxitron (HD Protocol), RS-485 |
| Video Outputs | AHD, Composite (FBAS) |
| Parts supplied |  |
| Scope of delivery | manual,plug set,bag of screws |
| Lens |  |
| Horizontal angle of view | 55,5 - 3° |
| Vertical angle of view | 32,4 - 1,7° |
| Focal length | 4,7 - 94 mm |
| Aperture Range (F) | F1.6~F3.0 |
| Lens Type | Auto Focus Zoom |
| Zoom Factor (Digital) | 16 |
| Zoomfactor (optical) | 20 |
| Video |  |
| Signal Format | AHD @ 2MP, Composite (FBAS), HD-TVI @ 2MP |
| Product labeling: |  |
| EAN/GTIN | 4260229840351 |
| Manufacturer order number | 226625 |
| Series | eneo COAXIZE |
| Brand | eneo |

Additional Productinformation

Related products

Recommendations

Services