Specifications text

Article number

226887

Type

eneo MPD-68M2713P0A

Short description

1/2,8" HD Dome Fix, Day&Night 3840x2160, Infrared, WDR, 2,7-13,5mm, 12VDC, Indoor

Main Features

1/2.8" 8.64 Megapixel SONY STARVIS CMOS-Sensor

Resolution max. 3840x2160@15/12,5fps

Motorised varifocal lens 2,7~13,5mm

Separate outputs for digital and multi-signal

Multisignal: HD-TVI, AHD selectable, Composite SUB

Digital signal: EX-SDI, HD-SDI selectable

DOL-WDR by triple scan images at 30/25fps &

ROI (Region of Interest) WDR mode

DIS (Digital Image Stabilization) function

Wedge adapter included (10° inclination)

Scratch resistant hard coated bubble

Easy installation, 3-Axis gimbal

Other Features (Specifications)

| General |  |
| --- | --- |
| Heating | No |
| Lighting | Infrared |
| Dimensions (WxHxD) (extended) | 100x116mm |
| Illumination range | ca. 30m |
| Weight | 0.26 kg |
| Color (Housing) | white |
| Color (dome) | clear |
| Housing type | Inside |
| Housing material | Plastic |
| Power consumption | 8 W |
| Mounting type | Ceiling, Wall |
| Power supply | 12VDC |
| Temperature Range (Operating) | -10°C ~ +50°C |
| Wide Dynamic Range (WDR) | D-WDR |
| Camera |  |
| Backlight compensation | Yes |
| Low Speed Shutter | Yes |
| Recording sensor | CMOS |
| Chip size | 1/2,8" |
| Flickerless function | Yes |
| Camera shape | Minsters |
| Signal-to-noise ratio | 50 dB |
| Day/night mode | Automatic,Day,Night,extern (built in Sensor),night-color mode |
| White balance | Automatic,AUTOext,Preset,Manual |
| Photosensitivity | 0,06 Lux bei F1,6,40 IRE |
| System | True Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| Display |  |
| Image resolution max. | 3840x2160,2560x1440,1920x1080 |
| DORI |  |
| Watch (62px/m) | 33,9 - 79,9 m |
| Detect (125px/m) | 16,8 - 39,7 m |
| Identify (250px/m) | 8,3 - 19,2 m |
| Perceiving (25px/m) | 84,2 - 198 m |
| Functions |  |
| Motion detector | Yes |
| Digital Noise Reduction (DNR) | Yes |
| Turning range | 165 ° |
| Menu languages | Chinese, English, German, Japanese, Korean |
| Vertical tilt range | +/-60° |
| Notes |  |
| Remark | NDAA compliance under Section 889 |
| Interfaces / Inputs/ Outputs |  |
| External connections | 12VDC input (DC plug),video (BNC),connection cable approx.15cm |
| Interface Protocols | UTC (HD Signal Format) |
| Control Interfaces | Coaxitron (HD Protocol) |
| Video Outputs | AHD, Composite (FBAS), CVI |
| Parts supplied |  |
| Scope of delivery | manual,Video sub-out cable,drilling template,bag of screws |
| Lens |  |
| Horizontal angle of view | 105 - 45° |
| Vertical angle of view | 59 - 25° |
| Focal length | 2,7 - 13,5 mm |
| Aperture Range (F) | F1.6 |
| Lens Type | Varifocal (motorized) |
| Zoomfactor (optical) | 5 |
| Video |  |
| Signal Format | AHD @ 8MP, Composite (FBAS), CVI @ 8MP, EX-SDI @ 8MP, HD-SDI @ 2MP, HD-TVI @ 8MP |
| Product labeling: |  |
| Manufacturer order number | 226887 |
| Series | eneo KX-Serie,eneo COAXIZE,eneo Callisto Indoor |
| Brand | eneo |

Additional Productinformation

Related products

Recommendations

Services