Specifications text

Article number

226889

Type

MPD-78M2713M0A

Short description

1/2,8" HD Dome Fix, Day&Night 3840x2160, Infrared, WDR, 2,7-13,5mm, 12/24V, IP67

Main Features

1/2.8" 8.64 Megapixel SONY STARVIS CMOS-Sensor

Resolution max. 3840x2160@15/12,5fps

Motorised varifocal lens 2,7~13,5mm

Separate outputs for digital and multi-signal

Multisignal: HD-TVI, AHD selectable, Composite SUB

Digital signal: EX-SDI, HD-SDI selectable

DOL-WDR by triple scan images at 30/25fps &

ROI (Region of Interest) WDR mode

DIS (Digital Image Stabilization) function

IP67, IK10, 3-Axis gimbal

For wall or ceiling flush mount

IR Illumination (2 x SMD High Power LED's)

Other Features (Specifications)

| Audio support | No |
| --- | --- |
| Recording sensor | CMOS |
| Lighting | Infrared |
| Motion detector | No |
| Horizontal angle of view | 105 - 45° |
| Focal length | 2,7 - 13,5 mm |
| Chip size | 1/2,8" |
| Digital Noise Reduction (DNR) | Yes |
| Turning range | 0 ° |
| Dimensions (WxHxD) (extended) | 119x140mm |
| Illumination range | ca. 40m |
| Remark | NDAA compliance under Section 889 |
| Aperture Range (F) | F1.6 |
| External connections | voltage input,video (BNC),connection cable approx.15cm |
| Flickerless function | Yes |
| Camera shape | Minsters |
| Scope of delivery | manual,Video sub-out cable,drilling template,bag of screws |
| Menu languages | Chinese, English, German, Japanese, Korean |
| Multicast | No |
| Interface Protocols | UTC (HD Signal Format) |
| Signal-to-noise ratio | 50 dB |
| Signal Format | AHD @ 8MP, Composite (FBAS), CVI @ 8MP, EX-SDI @ 8MP, HD-SDI @ 2MP, HD-TVI @ 8MP |
| Sunshade canopy | No |
| Day/night mode | Automatic,Day,Night,extern (built in Sensor),night-color mode |
| Vertical tilt range | 65 °(25 ~ 90°) |
| Integrated wall mount | No |
| White balance | Automatic,AUTOext,Preset,Manual |
| Color (dome) | clear |
| Backlight compensation | No |
| Housing type | Outside |
| Housing material | Aluminium |
| Heating | Yes |
| Photosensitivity | 0,06 Lux bei F1,6,40 IRE |
| Low Speed Shutter | No |
| Fan | No |
| Mounting type | Ceiling, Wall |
| Lens Type | Varifocal (motorized) |
| Manufacturer order number | 226889 |
| Protection | IP67 |
| Series | eneo KX-Serie,eneo COAXIZE,eneo Callisto |
| Control Interfaces | Coaxitron (HD Protocol) |
| Power supply | 12VDC, 24VAC |
| System | True Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| Temperature Range (Operating) | -50°C ~ +50°C |
| Vandalism-resistant | Yes |
| Wide Dynamic Range (WDR) | D-WDR |
| Certifications | IK10 |
| Zoomfactor (optical) | 5 |
| Brand | eneo |

Additional Productinformation

Related products

Recommendations

Services