Specifications text

Article number

228224

Type

IAM-5SE1004MUD

Short description

Switch, Unmanaged, 120W, 4xPoE 1x RJ45, 1x SFP 100Mbps, Industrial Design, -40°C~+75°C

Main Features

Ethernet PoE Switch, DIN Rail

Uplink ports 1xRJ45, 1xSFP

4x 10/100 PoE Ports,IEEE 802.3af/at

Up to 30W per port

Surge protection IEC61000-4-5 (6KV)

Status LED: Uplink, PoE, Optical port, Power

CCTV mode allows transmission up to 250m @ 10Mbps

Temperature range -40° C ~ +75° C

Other Features (Specifications)

| Number of PoE ports | 4 |
| --- | --- |
| Design | DIN Rail Module |
| Dimensions (WxHxD) (extended) | 46x110x159mm |
| Application environment | inside |
| ESD protection | IEC61000-4-2, IEC61000-4-5 |
| Weight | 530 kg |
| Scope of delivery | manual,power cord,Mounting Bracket,PSU |
| Multicast | No |
| Eneo _netzwerkstandards | IEEE 802.3 10BaseT, IEEE 802.3u 100BaseTX |
| Security | CCTV mode |
| Internal Switching Capacity (Gbit) | 1.5 Gbps |
| Internal Switching Capacity (MP per second) | 0.9 Mpps |
| Ethernet interface | 100Base-TX, 10Base-T |
| Type | Unmanaged |
| Fan | No |
| Management | No |
| Mounting type | DIN Rail, Wall |
| Manufacturer order number | 228224 |
| PoE Budget (Total) | 120 W |
| Ports | 1x 10/100Base-TX, 1x 100 SFP, 4x 10/100Base-TX |
| Power over Ethernet | IEEE 802.3af, IEEE 802.3at |
| Rack installation | No |
| Series | eneo Switch |
| Power supply | 48-57 VDC |
| Temperature Range (Operating) | -40°C ~ +75°C |
| VLAN | No |
| Brand | eneo |

Additional Productinformation

Related products

Recommendations

Services