Specifications text

Article number

227050

Type

eneo IPD-68M3611P5A

Short description

1/2,5" Network Dome, Fix, 3840x2160, Day&Night, DOL-WDR, 3,6-11mm, Infrared, Indoor

Main Features

1/2.5" Sony STARVIS CMOS sensor

Resolution max. 3840x2160, 8 Megapixel

Motorised varifocal lens 3.6~11mm

DOL-WDR (120dB) by triple scan images at 30/25fps

VCA video analysis, ONVIF event mapping

Browser independent (IE, Edge, Safari, Firefox, Chrome)

Smart Codec H.265 ROI (Region of interest)

Quad Smart Streaming: H.265(+), H.264(+), MJPEG

Slot for Micro SD Card, HTML5 replay

Power supply PoE or 12VDC

Wedge adapter included (10° inclination)

Supports ONVIF Profile S and Profile T

Other Features (Specifications)

| General |  |
| --- | --- |
| Heating | No |
| Vandalism-resistant | Yes |
| Lighting | Infrared |
| Dimensions (WxHxD) (extended) | 94x116mm |
| Weight | 0.3 kg |
| SDK | upon request |
| Text overlay | available |
| Color (Housing) | white |
| Color (dome) | clear |
| Housing type | Inside |
| Housing material | Plastic |
| Power consumption | 8.5 W |
| Mounting type | Ceiling, Wall |
| ONVIF | ONVIF Profile S, ONVIF Profile T |
| Power over Ethernet | IEEE 802.3af |
| Power supply | 12VDC, Poe |
| Temperature Range (Operating) | -10°C ~ +50°C |
| Wide Dynamic Range (WDR) | D-WDR |
| Camera |  |
| Backlight compensation | Yes |
| Low Speed Shutter | Yes |
| Resolution standard | 8 MP |
| Recording sensor | CMOS |
| Chip size | 1/2,5" |
| Flickerless function | Yes |
| Camera shape | Minsters |
| Signal-to-noise ratio | 50 dB |
| Day/night mode | Automatic,Day,Night,night-color mode,extern (Sensor) |
| White balance | ATW-Indoor,ATW-Outdoor,Manual,Fluorescent light,Light bulb,Shade,Clear Sky,Flammes |
| Photosensitivity | 0,2 Lux bei F1,5,40 IRE |
| System | True Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| Display |  |
| Image resolution max. | 3840x2160 |
| DORI |  |
| Watch (62px/m) | 36,7 - 85,7 m |
| Detect (125px/m) | 17,8 - 42,7 m |
| Identify (250px/m) | 8,6 - 21,2 m |
| Perceiving (25px/m) | 91,6 - 214,2 m |
| Functions |  |
| Audio support | No |
| Motion detector | Yes |
| Digital Noise Reduction (DNR) | Yes |
| Multicast | Yes |
| Analysis | Motion Detection |
| Turning range | 165 ° |
| Alarm processing | recording internal storage (SD card),E-Mail,HTTP,FTP,Boost,TCP,UDP,Onvif Event Mapping |
| Browser | Edge: MJPEG, Mozilla: MJPEG, Safari: MJPEG |
| Menu languages | English, French, German, Korean |
| Password | Administrator and User |
| Vertical tilt range | +/-60° |
| Internal memory | microSDHC card, microSDXC card |
| Interfaces / Inputs/ Outputs |  |
| External connections | RJ-45,12VDC input (DC plug),connection cable approx.55cm |
| Control Interfaces | Ethernet |
| Parts supplied |  |
| Scope of delivery | manual,bag of screws |
| Lens |  |
| Horizontal angle of view | 84 - 40° |
| Vertical angle of view | 46 - 21° |
| Focal length | 3,6 - 11 mm |
| MOD (Minimum Object Distance) | 0.5 m |
| Lens Type | Varifocal (motorized) |
| Zoomfactor (optical) | 3 |
| Video |  |
| Image transfer rate max. | 30 fps |
| Resolution (extended) | 3840x2160 (16:9),2592x1520 (16:9),1920x1080 (16:9),1600x900 (16:9),1280x720 (16:9),960x540 (16:9),640x360 (16:9),320x180 (16:9) |
| Data stream types | Multi-Streaming |
| Video Compression | H.264, H.264+, H.265, H.265+, M-JPEG |
| Product labeling: |  |
| Manufacturer order number | 227050 |
| Series | eneo LN-Serie |
| Brand | eneo |

Additional Productinformation

Related products

| eneo Site Manager | Management Software for eneo IP products |
| --- | --- |

Recommendations

Services

| PAINTING SERVICE basic costs | once per RAL color and delivery lot |
| --- | --- |
| PAINTING SERVICE fix dome camera | in RAL color |
| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |