Specifications text

Article number

227053

Type

eneo IPD-65M2713P5A

Short description

1/2,8" Network Dome, Fix, 2592x1944, Day&Night, DOL-WDR, 2,7-13,5mm, Infrared, Indoor

Main Features

1/2.8" Sony STARVIS CMOS sensor

Resolution max. 2592x1944, 5 Megapixel

Motorised varifocal lens 2,7~13.5mm

DOL-WDR (120dB) by triple scan images at 30/25fps

VCA (Video Content Analysis), ONVIF event mapping

Browser independent (IE, Edge, Safari, Firefox, Chrome)

Smart Codec H.265 ROI (Region of interest)

Quad Smart Streaming: H.265(+), H.264(+), MJPEG

Micro SD Card, HTML5 replay

Power supply PoE or 12VDC

Wedge adapter included (10° inclination)

Supports ONVIF Profile S and Profile T

Other Features (Specifications)

| General |  |
| --- | --- |
| Heating | No |
| Vandalism-resistant | Yes |
| Lighting | Infrared |
| Dimensions (WxHxD) (extended) | 94x116mm |
| Weight | 0.3 kg |
| SDK | upon request |
| Text overlay | available |
| Color (Housing) | white |
| Color (dome) | clear |
| Housing type | Inside |
| Housing material | Plastic |
| Power consumption | 8.5 W |
| Mounting type | Ceiling, Wall |
| ONVIF | ONVIF Profile S, ONVIF Profile T |
| Power over Ethernet | IEEE 802.3af |
| Power supply | 12VDC, Poe |
| Temperature Range (Operating) | -10°C ~ +50°C |
| Wide Dynamic Range (WDR) | D-WDR |
| Camera |  |
| Backlight compensation | Yes |
| Low Speed Shutter | Yes |
| Resolution standard | 5 MP |
| Recording sensor | CMOS |
| Chip size | 1/2,8" |
| Flickerless function | Yes |
| Camera shape | Minsters |
| Signal-to-noise ratio | 50 dB |
| Day/night mode | Automatic,Day,Night,night-color mode,extern (Sensor) |
| White balance | ATW-Indoor,ATW-Outdoor,Manual,Fluorescent light,Light bulb,Shade,Clear Sky,Flammes |
| Photosensitivity | 0,05 Lux bei F1,6,40 IRE |
| System | True Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| Display |  |
| Image resolution max. | 2592x1944 |
| DORI |  |
| Watch (62px/m) | 16,3 - 69,1 m |
| Detect (125px/m) | 7,7 - 34,2 m |
| Identify (250px/m) | 3,6 - 17 m |
| Perceiving (25px/m) | 41,7 - 171,7 m |
| Functions |  |
| Analysis | Motion Detection |
| Audio support | No |
| Motion detector | Yes |
| Digital Noise Reduction (DNR) | Yes |
| Multicast | Yes |
| Turning range | 165 ° |
| Alarm processing | recording internal storage (SD card),E-Mail,HTTP,FTP,Boost,TCP,UDP,Onvif Event Mapping |
| Browser | Edge: MJPEG, Mozilla: MJPEG, Safari: MJPEG |
| Menu languages | English, French, German, Korean |
| Password | Administrator and User |
| Vertical tilt range | +/-60° |
| Internal memory | microSDHC card, microSDXC card |
| Interfaces / Inputs/ Outputs |  |
| External connections | micro SD card slot,RJ-45,12VDC power input,connection cable approx.55cm |
| Control Interfaces | Ethernet |
| Parts supplied |  |
| Scope of delivery | manual,bag of screws |
| Lens |  |
| Horizontal angle of view | 97 - 29° |
| Vertical angle of view | 54 - 17° |
| Focal length | 2,7 - 13,5 mm |
| MOD (Minimum Object Distance) | 0.5 m |
| Lens Type | Varifocal (motorized) |
| Zoomfactor (optical) | 4 |
| Video |  |
| Image transfer rate max. | 30 fps |
| Resolution (extended) | 2592x1944 (4:3),2592x1520 (16:9),2560x1440 (16:9),1920x1080 (16:9),1600x900 (16:9),1280x720 (16:9),960x540 (16:9),640x360 (16:9),320x180 (16:9) |
| Data stream types | Multi-Streaming |
| Video Compression | H.264, H.264+, H.265, H.265+, M-JPEG |
| Product labeling: |  |
| Manufacturer order number | 227053 |
| Series | eneo Callisto Indoor,eneo LN-Serie |
| Brand | eneo |

Additional Productinformation

Related products

Recommendations

Services