Specifications text

Article number

220930

Type

IPD-78M3611M0A

Short description

1/2,5" Network Dome, Fix, 3840x2160, Day&Night, DOL-WDR 3,6-11mm, Infrared, IP67

Main Features

1/2.5" Sony STARVIS CMOS sensor

Resolution max. 3840x2160, 8 Megapixel

Motorised varifocal lens 3,6~11mm

DOL-WDR (120dB) by triple scan images at 30/25fps

VCA video analysis, ONVIF event mapping

Browser independent (IE, Edge, Safari, Firefox, Chrome)

Smart Codec H.265 ROI (Region of interest)

Quad Smart Streaming: H.265(+), H.264(+), MJPEG

Slot for Micro SD Card, HTML5 replay, Audio, Alarm

Power supply PoE, 12VDC or 24VAC

For outdoor use IP66, vandal proofed IK10

Supports ONVIF Profile S and Profile T

Other Features (Specifications)

| General |  |
| --- | --- |
| Vandalism-resistant | Yes |
| Lighting | Infrared |
| Dimensions (WxHxD) (extended) | 119x140mm |
| SDK | upon request |
| Text overlay | available |
| Color (dome) | clear |
| Housing type | Inside, Outside |
| Housing material | Aluminium |
| Power consumption | 10.8 W |
| Mounting type | Ceiling, Wall |
| ONVIF | ONVIF Profile S, ONVIF Profile T |
| Power over Ethernet | IEEE 802.3af |
| Protection | IP67 |
| Power supply | 12VDC, 24VAC, Poe |
| Temperature Range (Operating) | -50°C ~ +50°C |
| Wide Dynamic Range (WDR) | D-WDR |
| Certifications | IK10 |
| Zoomfactor (optical) | 3 |
| Camera |  |
| Resolution standard | 8 MP |
| Recording sensor | CMOS |
| Chip size | 1/2,5" |
| Flickerless function | Yes |
| Camera shape | Minsters |
| Signal-to-noise ratio | 50 dB |
| Day/night mode | Automatic,Day,Night,night-color mode,extern (Sensor) |
| White balance | ATW-Indoor,ATW-Outdoor,Manual,Fluorescent light,Light bulb,Shade,Clear Sky,Flammes |
| Photosensitivity | 0,2 Lux bei F1,5,40 IRE |
| System | True Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| Display |  |
| Image resolution max. | 3840x2160 |
| DORI |  |
| Watch (62px/m) | 40.4 m |
| Detect (125px/m) | 98 m |
| Identify (250px/m) | 98 m |
| Perceiving (25px/m) | 104 m |
| Functions |  |
| Audio support | Yes |
| Digital Noise Reduction (DNR) | Yes |
| Multicast | Yes |
| Turning range | 173 ° |
| Alarm processing | recording internal storage (SD card),E-Mail,HTTP,FTP,Boost,TCP,UDP,Onvif Event Mapping,alarm out |
| Audio | 2 Way Audio |
| Browser | Edge: MJPEG, Mozilla: MJPEG, Safari: MJPEG |
| Menu languages | English, French, German, Korean |
| Password | Administrator and User |
| Vertical tilt range | 65 °(25 ~ 90°) |
| Internal memory | microSDHC card, microSDXC card |
| Interfaces / Inputs/ Outputs |  |
| Alarm inputs | 1 |
| External connections | micro SD card slot,Alarm in/output,RJ-45,Combi connector,connection cable approx.15cm |
| Control Interfaces | Ethernet |
| Parts supplied |  |
| Scope of delivery | manual,bag of screws |
| Lens |  |
| Horizontal angle of view | 84 - 40° |
| Focal length | 3,6 - 11 mm |
| MOD (Minimum Object Distance) | 0.5 m |
| Lens Type | Varifocal (motorized) |
| Video |  |
| Image transfer rate max. | 30 fps |
| Resolution (extended) | 3840x2160 (16:9),2592x1520 (16:9),1920x1080 (16:9),1600x900 (16:9),1280x720 (16:9),960x540 (16:9),640x360 (16:9),320x180 (16:9) |
| Data stream types | Multi-Streaming |
| Video Compression | H.264, H.264+, H.265, H.265+, M-JPEG |
| Product labeling: |  |
| Manufacturer order number | 220930 |
| Series | eneo Callisto,eneo LN-Serie |
| Brand | eneo |

Additional Productinformation

Related products

| AK-5 | Camera Junction Box, for Callisto Dome series, IP66 |
| --- | --- |
| DOME-S1 | Sun Shield for eneo Callisto Dome Series |
| DOME-WB1+DOME/ Basis1 | Wall Bracket & Surface mount base, Callisto Dome line |
| DOME-WB3 | Wall bracket for eneo Dome Cameras MED-62xxxxxM0A and IED-62xxxxxM0A |
| DOME/BASIS1 | Surface mount base , Callisto Dome line |
| DOME/WEDGE2 | Wedge Adapter, Callisto Dome Line |
| eneo Site Manager | Management Software for eneo IP products |
| IAM-4MU1001M0A | EPOC Signal Converter, BNC to 2-wire cable transmission for EPOC Cameras, Transceiver |

Recommendations

Services

| LACKIERUNG Fix-Dome-Kamera | Extra charge for special paint fixed dome in RAL color |
| --- | --- |
| LACKIERUNG Grundkosten | Painting basic costs once per RAL color and delivery lot |
| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |