Specifications text

Article number

226921

Type

IAR-7SG1008MMB

Short description

Gigabit Switch, Managed, 135W, 8x PoE, 2x SFP, 2x RJ45, 10/100/1000 Mbps, 19", 1 HE

Main Features

Other Features (Specifications)

| Brand | eneo |
| --- | --- |

Additional Productinformation

Related products

| RJT-1000SFP | RJ45- SFP Transceiver Modul, 1000Base-T, 0°~ +70°C |
| --- | --- |
| SFP-0850M10 | SFP Modul, 1.25Gbps, 550m, Multimode, 850nm, Duplex LC Connector, 0°~+70°C |
| SFP-0850M40 | SFP Modul, 1,25Gbps, 550m, Multimode, 850nm, Duplex LC Connector, -40° ~ +70°C |
| SFP-1310M40 | SFP Modul, 1.25Gbps, 550m, Multimode, 1310nm, Duplex LC Connector, -40°~+75°C |
| SFP-1310S10 | SFP Modul, 1.25Gbps, 20km, Singlemode, 1310nm, Duplex LC Connector, 0°~+70°C |
| SFP-1310S40 | SFP Modul, 1.25Gbps, 20km, Singlemode, 1310nm, Duplex LC Connector -40°~+75°C |

Recommendations

Services