Specifications text

Article number

226691

Type

eneo IAR-7SG1016MMB

Short description

Gigabit Switch, Managed 270W, 16x PoE, 4x SFP, 10/100/1000 Mbps, 19", 1 U

Main Features

PoE Switch, Rack Montage

Gigabit Uplink Ports: 4x SFP

16x 10/100/1000 PoE Ports, IEEE 802.3af/at

Up to 30W per port, total ≤270W

Fast Ring-Redundant, Reconfiguration

ESD protection IEC61000-4-5 standard

Managed L2 switch

Web configuration, Web management

VLAN, PoE management, MAC address table

Port mirroring, password

Temperature range -10° C ~ +50° C

Rack mounting brackets included

Other Features (Specifications)

| General |  |
| --- | --- |
| Fan | Yes |
| Management | Yes |
| Number of PoE ports | 16 |
| Design | Rack |
| Dimensions (WxHxD) (extended) | 44,5x442x320mm |
| Application environment | inside |
| ESD protection | IEC61000-4-2, IEC61000-4-5 |
| Weight | 4.5 kg |
| Type | Layer2 Switch |
| Power consumption | 320 W |
| Mounting type | 19" Rack Installation |
| PoE Budget (Total) | 270 W |
| Power over Ethernet | IEEE 802.3af, IEEE 802.3at |
| Power supply | 100-240VAC |
| Temperature Range (Operating) | -10°C ~ +50°C |
| Functions |  |
| Multicast | Yes |
| Interfaces / Inputs/ Outputs |  |
| Ethernet interface | 1000Base-T, 100Base-TX, 10Base-T |
| Ports | 16x 10/100/1000Base-T, 4x 1000Mbps SFP, Console RJ45-RS232, Console RJ45-RS232, OOB 10/100Base-TX, OOB 10/100Base-TX |
| Parts supplied |  |
| Scope of delivery | manual,power cord,19\" rack mount angle |
| Network |  |
| VLAN | Yes |
| Eneo _netzwerkstandards | IEEE 802.1D Spanning Tree Protocol, IEEE 802.3 10BaseT, IEEE 802.3u 100BaseTX, IEEE 802.3x Flow Control |
| Security | Fast Ring, Password, Port Isolation, Port Speed Setting, Radius Authentication |
| Internal Switching Capacity (Gbit) | 40 Gbps |
| Internal Switching Capacity (MP per second) | 29.8 Mpps |
| Product labeling: |  |
| Manufacturer order number | 226691 |
| Series | eneo Switch |
| Brand | eneo |

Additional Productinformation

Related products

Recommendations

Services