Specifications text

Article number

230114

Type

IAR-7SE2016MMA

Short description

Gigabit Switch, Managed, 250W 16x PoE, 4x SFP, 1000Mbps, Wall-/Rack mounting, Desktop

Main Features

16-Port Gigabit PoE L2 Managed Ethernet Switch

4x SFP Gigabit Uplink Ports

PoE Standard IEEE 802.3af/at (PSE)

Up to 30W per port, total ≤250W

Fast Ring-Redundant, Reconfiguration

Surge Immunity 6kV, Standard: IEC6000-4-5

ESD 6 kV Contact Discharge, 8kV Air Discharge, standard: IEC61000-4-2

Web configuration, Web management

VLAN, PoE management, MAC address table

Port mirroring, password protection

Temperature range -10° C ~ +50° C

Rack mounting brackets included

Other Features (Specifications)

| Dimensions (HxWxD) | 44x442x320mm |
| --- | --- |
| Type | Layer2 switch |
| Series | eneo Switch |
| Mounting | 19" rack, desktop |
| Blower | yes |
| Ethernet interface | 1000Base-T, 100Base-TX, 10Base-T |
| VLAN | yes |
| Ports | 16x 10/100/1000Base-T, 4x 1000Mbps SFP, Console RJ45-RS232 |
| PoE ports (number) | 16 |
| Management | yes |
| Model | Desktop, Rack Mount |
| Supply voltage | 100~240VAC |
| Power consumption | 270W |
| Power over Ethernet | IEEE 802.3af, IEEE 802.3at |
| PoE budget | 250 W |
| Operating temperature | -10°C ~ +50°C |
| Weight | 1.9kg |
| Manufacturer order number | 230114 |
| Brand | eneo |

Additional Productinformation

Related products

| eneo Site Manager | Management Software for eneo IP products |
| --- | --- |
| SFP-0850M10 | SFP Modul, 1.25Gbps, 550m, Multimode, 850nm, Duplex LC Connector, 0°~+70°C |
| SFP-0850M40 | SFP Modul, 1,25Gbps, 550m, Multimode, 850nm, Duplex LC Connector, -40° ~ +70°C |
| SFP-0850MM10 | SFP Modul, 155Mbps , 2km, Multimode, 850nm, Duplex LC Connector, 0°~+70°C |
| SFP-1310M40 | SFP Modul, 1.25Gbps, 550m, Multimode, 1310nm, Duplex LC Connector, -40°~+75°C |
| SFP-1310S10 | SFP Modul, 1.25Gbps, 20km, Singlemode, 1310nm, Duplex LC Connector, 0°~+70°C |
| SFP-1310S40 | SFP Modul, 1.25Gbps, 20km, Singlemode, 1310nm, Duplex LC Connector -40°~+75°C |

Recommendations

Services