Specifications text

Article number

230033

Type

IAR-7SE1024MMC

Short description

Gigabit Switch managed, 350W Layer 3, 24xPoE, 4x SFP 10Gb, Wall-, Rackmount, Desktop

Main Features

24-Port Gigabit PoE L3 Managed Ethernet Switch

4x SFP 10 Gigabit Uplink Ports

PoE Standard IEEE 802.3af/at (PSE)

Up to 30W per port, total ≤350W

Fast Ring-Redundant, Reconfiguration

Surge Immunity 6kV, Standard: IEC6000-4-5

ESD 6 kV Contact Discharge, 8kV Air Discharge, standard: IEC61000-4-2

Web configuration, Web management

VLAN, PoE management, MAC address table

Port mirroring, password protection

Temperature range -10° C ~ +50° C

Rack mounting brackets included

Other Features (Specifications)

| Number of PoE ports | 24 |
| --- | --- |
| Design | Desktop, Rack |
| Dimensions (WxHxD) (extended) | 44,5x442x320mm |
| ESD protection | IEC61000-4-2, IEC61000-4-5 |
| Weight | 6.5 kg |
| Scope of delivery | manual,power cord,19\" rack mount angle |
| Multicast | Yes |
| Eneo _netzwerkstandards | IEEE 802.1D Spanning Tree Protocol, IEEE 802.1w Rapid Spanning Tree Protocol, IEEE 802.3 10BaseT, IEEE 802.3ad Link Aggregation, IEEE 802.3u 100BaseTX, IEEE 802.3x Flow Control |
| Security | Fast Ring, Password, Port Isolation, Port Speed Setting, Radius Authentication |
| Internal Switching Capacity (Gbit) | 128 Gbps |
| Internal Switching Capacity (MP per second) | 92.5 Mpps |
| Ethernet interface | 1000Base-T, 100Base-TX, 10Base-T |
| Type | Layer3 Switch |
| Power consumption | 380 W |
| Fan | No |
| Management | Yes |
| Mounting type | 19" Rack Installation, Desktop |
| Manufacturer order number | 230033 |
| PoE Budget (Total) | 350 W |
| Ports | 24x 10/100/1000Base-T, 4x 10GBit SFP, Console RJ45-RS232, Console RJ45-RS232, OOB 10/100Base-TX, OOB 10/100Base-TX |
| Power over Ethernet | IEEE 802.3af, IEEE 802.3at |
| Rack installation | No |
| Series | eneo Switch |
| Power supply | 100~240VAC |
| Temperature Range (Operating) | -10°C ~ +50°C |
| VLAN | Yes |
| Brand | eneo |

Additional Productinformation

Related products

Recommendations

Services