Specifications text

Article number

230034

Type

eneo IAM-5SE1008IUA

Short description

Switch unmanged, 120W, 8xPoE 1xRJ45 ,1xSFP Gigabit Industrial Design

Main Features

Ethernet PoE Switch, DIN Rail

Gigabit Uplink ports 1xRJ45, 1xSFP

8x 10/100 PoE Ports,IEEE 802.3af/at

Up to 30W per port, Total ≤240W (optional power supply)

Surge protection IEC61000-4-5 (6KV)

Status LED: Uplink, PoE, Optical port, Power

CCTV mode allows transmission up to 250m @ 10Mbps

Temperature range -30° C ~ +65° C

Other Features (Specifications)

| General |  |
| --- | --- |
| Number of PoE ports | 8 |
| Design | Case |
| Dimensions (WxHxD) (extended) | 44x159x110mm |
| Application environment | inside |
| ESD protection | IEC61000-4-2, IEC61000-4-5 |
| Weight | 500 g |
| Type | Unmanaged |
| Mounting type | Desktop |
| PoE Budget (Total) | 240 W |
| Power over Ethernet | IEEE 802.3af, IEEE 802.3at |
| Power supply | 48-57 VDC |
| Temperature Range (Operating) | -30°C ~ +65°C |
| Functions |  |
| Menu languages | English, German |
| Notes |  |
| Remark | External power supply specification: 100~240VAC,120W,-10°~+55°.,To achieve the specified PoE budget (total) value,an appropriate optional power supply unit must be used. |
| Interfaces / Inputs/ Outputs |  |
| Ethernet interface | 1000Base-T, 100Base-TX, 10Base-T |
| PoE Standard | IEEE802.3af, IEEE802.3at |
| Ports | 1x 10/100/1000Base-T, 1x 10/100/1000T SFP, 8x 10/100Base-TX |
| Parts supplied |  |
| Scope of delivery | manual,power cord,Mounting Bracket,PSU |
| Network |  |
| VLAN | Yes |
| Eneo _netzwerkstandards | IEEE 802.3 10BaseT, IEEE 802.3u 100BaseTX, IEEE 802.3x Flow Control |
| Security | CCTV mode |
| Internal Switching Capacity (Gbit) | 5.6 Gbps |
| Internal Switching Capacity (MP per second) | 4.0 Mpps |
| Product labeling: |  |
| Manufacturer order number | 230034 |
| Series | eneo Switch |
| Brand | eneo |

Additional Productinformation

Related products

Recommendations

Services