Specifications text

Article number

228223

Type

eneo IAM-5SE1008MUB

Short description

Gigabit Switch, 8xPoE 100Mbps, 1x RJ45, 1x SFP 1000Mbps, Unmanaged, Industrial Design

Main Features

Gigabit PoE switch, DIN Rail

Gigabit Uplink ports 1xRJ45, 1xSFP

8x 10/100 PoE Ports, IEEE 802.3af/at

Up to 30W per port

Surge protection IEC61000-4-5 (6KV)

Status LED: Uplink, PoE, Optical port, Power

PoE reset button for each port

CCTV mode allows transmission up to 250m @ 10Mbps

Temperature range -40° C ~ +75° C

Other Features (Specifications)

| General |  |
| --- | --- |
| Number of PoE ports | 8 |
| Design | DIN Rail Module |
| Dimensions (WxHxD) (extended) | 46,5x163x110,5mm |
| Application environment | inside |
| ESD protection | IEC61000-4-2, IEC61000-4-5 |
| Weight | 530 kg |
| Type | Unmanaged |
| Mounting type | DIN Rail, Wall |
| PoE Budget (Total) | 120 W |
| Power over Ethernet | IEEE 802.3af, IEEE 802.3at |
| Power supply | 48-57 VDC |
| Temperature Range (Operating) | -40°C ~ +75°C |
| Interfaces / Inputs/ Outputs |  |
| Ethernet interface | 1000Base-T, 100Base-TX, 10Base-T |
| Ports | 1x 10/100/1000Base-TX, 1x 10/100/1000T SFP, 8x 10/100Base-TX |
| Parts supplied |  |
| Scope of delivery | manual,power cord,Mounting Bracket,PSU |
| Network |  |
| VLAN | Yes |
| Eneo _netzwerkstandards | IEEE 802.3 10BaseT, IEEE 802.3u 100BaseTX |
| Internal Switching Capacity (Gbit) | 5.6 Gbps |
| Internal Switching Capacity (MP per second) | 4.2 Mpps |
| Product labeling: |  |
| Manufacturer order number | 228223 |
| Series | eneo Switch |
| Brand | eneo |

Additional Productinformation

Related products

| eneo SFP-0850M10 | SFP Modul, 1.25Gbps, 550m, Multimode, 850nm, Duplex LC Connector, 0°~+70°C |
| --- | --- |
| eneo SFP-0850M40 | SFP Modul, 1,25Gbps, 550m, Multimode, 850nm, Duplex LC Connector, -40° ~ +70°C |
| eneo SFP-1310M40 | SFP Modul, 1.25Gbps, 550m, Multimode, 1310nm, Duplex LC Connector, -40°~+75°C |
| eneo SFP-1310S10 | SFP Modul, 1.25Gbps, 20km, Singlemode, 1310nm, Duplex LC Connector, 0°~+70°C |
| eneo SFP-1310S40 | SFP Modul, 1.25Gbps, 20km, Singlemode, 1310nm, Duplex LC Connector -40°~+75°C |

Recommendations

| jetrics NDR-240-48 DIN RAIL | Power Supply, 230VAC / 48VDC, max. 240W, for DIN rail mounting |
| --- | --- |

Services