Specifications text

Article number

230036

Type

eneo IAM-5SE1016IMA

Short description

Gigabit Switch, Managed, 360W Layer 3, 16xPoE, 4x SFP 10G, Industrial Design

Main Features

16-Port Gigabit Switch Managed L3, Fanless

4x SFP 10 Gigabit Uplink Ports

PoE Standard IEEE 802.3af/at

Up to 30W per port, total ≤360W

Power supply not included

Fast Ring-Redundant, Reconfiguration

ESD protection IEC61000-4-5 standard

Web configuration, Web management

VLAN, PoE management, MAC address table

Port mirroring, password protection

Relay Alarm for power, cable link and PoE link down

Temperature range -40° C ~ +75° C

Other Features (Specifications)

| General |  |
| --- | --- |
| Management | Yes |
| Number of PoE ports | 16 |
| Design | DIN Rail Module |
| Dimensions (WxHxD) (extended) | 95,6x163x110mm |
| ESD protection | IEC61000-4-2, IEC61000-4-5 |
| Weight | 1.7 kg |
| Type | Layer3 Switch |
| Power consumption | 380 W |
| Mounting type | DIN Rail, Wall |
| PoE Budget (Total) | 360 W |
| Power over Ethernet | IEEE 802.3af, IEEE 802.3at |
| Power supply | 48-57 VDC |
| Temperature Range (Operating) | -40°C ~ +75°C |
| Functions |  |
| Multicast | Yes |
| Notes |  |
| Remark | Power supply unit is not included in the scope of delivery,To achieve the specified PoE budget (total) value,an appropriate optional power supply unit must be used. |
| Interfaces / Inputs/ Outputs |  |
| Ethernet interface | 1000Base-T, 100Base-TX, 10Base-T |
| Ports | 16x 10/100/1000Base-T, 4x 10GBit SFP, Console RJ45-RS232, Console RJ45-RS232, OOB 10/100Base-TX, OOB 10/100Base-TX |
| Parts supplied |  |
| Scope of delivery | manual,Mounting Bracket |
| Network |  |
| VLAN | Yes |
| Eneo _netzwerkstandards | IEEE 802.1D Spanning Tree Protocol, IEEE 802.1w Rapid Spanning Tree Protocol, IEEE 802.3 10BaseT, IEEE 802.3ad Link Aggregation, IEEE 802.3u 100BaseTX, IEEE 802.3x Flow Control |
| Security | Fast Ring, Password, Port Isolation, Port Speed Setting, Radius Authentication |
| Internal Switching Capacity (Gbit) | 112 Gbps |
| Product labeling: |  |
| Manufacturer order number | 230036 |
| Series | eneo Switch |
| Brand | eneo |

Additional Productinformation

Related products

Recommendations

Services