Specifications text

Article number

230035

Type

eneo IAM-6SE1008IMA

Short description

Gigabit Switch, Managed, 240W, Layer 3, 8xPoE, 4x SFP 10G, Industrial Design

Main Features

8-Port Gigabit Switch Managed L3, Fanless

4x SFP 10 Gigabit Uplink Ports

PoE Standard IEEE 802.3af/at

Up to 30W per port, total ≤240W (Power supply not included)

Fast Ring-Redundant, Reconfiguration

ESD protection IEC61000-4-5 standard

Web configuration, Web management

VLAN, PoE management, MAC address table

Port mirroring, password protection

Relay Alarm for power failure, cable link down and PoE link down

Temperature range -40° C ~ +75° C

DIN-rail, Rack mounting brackets included

Other Features (Specifications)

| General |  |
| --- | --- |
| Management | Yes |
| Number of PoE ports | 8 |
| Design | DIN Rail Module |
| Dimensions (WxHxD) (extended) | 95,6x163x110mm |
| ESD protection | IEC61000-4-2, IEC61000-4-5 |
| Weight | 1.9 kg |
| Type | Layer3 Switch |
| Power consumption | 260 W |
| Mounting type | DIN Rail, Wall |
| PoE Budget (Total) | 240 W |
| Power over Ethernet | IEEE 802.3af, IEEE 802.3at |
| Power supply | 48-57 VDC |
| Temperature Range (Operating) | -40°C ~ +75°C |
| Functions |  |
| Multicast | Yes |
| Notes |  |
| Remark | Power supply unit is not included in the scope of delivery,To achieve the specified PoE budget (total) value,an appropriate optional power supply unit must be used. |
| Interfaces / Inputs/ Outputs |  |
| Ethernet interface | 1000Base-T, 100Base-TX, 10Base-T |
| Ports | 4x 10GBit SFP, 8x 10/100/1000Base-T, Console RJ45-RS232, Console RJ45-RS232 |
| Parts supplied |  |
| Scope of delivery | manual,Mounting Bracket |
| Network |  |
| VLAN | Yes |
| Eneo _netzwerkstandards | IEEE 802.1D Spanning Tree Protocol, IEEE 802.1w Rapid Spanning Tree Protocol, IEEE 802.3 10BaseT, IEEE 802.3ad Link Aggregation, IEEE 802.3u 100BaseTX, IEEE 802.3x Flow Control |
| Security | Fast Ring, Password, Port Isolation, Port Speed Setting, Radius Authentication |
| Internal Switching Capacity (Gbit) | 96 Gbps |
| Product labeling: |  |
| Manufacturer order number | 230035 |
| Series | eneo Switch |
| Brand | eneo |

Additional Productinformation

Related products

| eneo SFP-0850M10 | SFP Modul, 1.25Gbps, 550m, Multimode, 850nm, Duplex LC Connector, 0°~+70°C |
| --- | --- |
| eneo SFP-0850M10-10G | SFP Modul, 10Gbps, 300m, Multimode, 850nm, Duplex LC Connector, -5°~+70°C |
| eneo SFP-0850M40 | SFP Modul, 1,25Gbps, 550m, Multimode, 850nm, Duplex LC Connector, -40° ~ +70°C |
| eneo SFP-1310M40 | SFP Modul, 1.25Gbps, 550m, Multimode, 1310nm, Duplex LC Connector, -40°~+75°C |
| eneo SFP-1310S10 | SFP Modul, 1.25Gbps, 20km, Singlemode, 1310nm, Duplex LC Connector, 0°~+70°C |
| eneo SFP-1310S10-10G | SFP Modul, 10Gbps, 10km, Singlemode, 1310nm, Duplex LC Connector, -5°~+70°C |
| eneo SFP-1310S40 | SFP Modul, 1.25Gbps, 20km, Singlemode, 1310nm, Duplex LC Connector -40°~+75°C |
| eneo Site Manager | Management Software for eneo IP products |

Recommendations

| jetrics NDR-240-48 DIN RAIL | Power Supply, 230VAC / 48VDC, max. 240W, for DIN rail mounting |
| --- | --- |

Services