Specifications text

Article number

229200

Type

eneo VM-HD19MR

Short description

19" (48cm) LCD, industrial Monitor, 1280x1024, Protection Glass, Rack Mount, 12V, HDMI

Main Features

Professional 19" (48 cm) LCD monitor

Built to run 24x7x365 continuously (Industrial Level)

Including protection glass, Anti-Glare-Display 3H

Extended temperature range -30°C +85°C

Maximum resolution 1280x1024 pixels

LED Back Light

Fast response time 5ms

Different video inputs HDMI, VGA, BNC (CVBS)

Audio In- and Output

Built-in speakers

Supply voltage: 12VDC

Including 19" mounting frame

Other Features (Specifications)

| General |  |
| --- | --- |
| Applications | Industry |
| Dimensions (WxHxD) (extended) | 482x355x48mm |
| Fortification | VESA Standard 100x100mm, VESA Standard 75x75mm |
| Weight | 3 kg |
| Energy efficiency class | E |
| Color (Housing) | Black |
| Housing material | Metal |
| Power consumption | 20 W |
| Power supply | 12VDC |
| Display |  |
| Viewing Angle Horizontal | 178 ° |
| Viewing Angle Vertical | 178 ° |
| Screen diagonal (cm) | 48 cm |
| Screen diagonal (inches) | 19 " |
| Display | Anti-glare display, Color TFT-LCD, Hard coating (3H) |
| Pixel error class | ISO 13406-2, Class 2 |
| Brightness (typical) | 250 cd/m² |
| Backlight | LED |
| Contrast ratio (typical) | 1000:1 |
| Response time (typical) | 5 ms |
| Aspect ratio | 5:4 |
| Refresh rate | 60 Hz |
| Functions |  |
| Comb filter | Yes |
| Menu languages | English |
| Notes |  |
| Remark | NDAA compliance under Section 889 |
| Interfaces / Inputs/ Outputs |  |
| Audio Inputs | 1 |
| Transmission Technology | HDCP 1.4, HDCP 2.2, HDMI 1.4a |
| Video Outputs | Composite (FBAS) |
| Video Inputs | 2x Composite (FBAS), HDMI, VGA |
| Parts supplied |  |
| Scope of delivery | manual,power cord,PSU 100~240VAC,IR remote controller,HDMI cable |
| Video |  |
| Resolution (extended) | 800x600,1024x768,1280x1024 |
| Maximum Image Resolution (HV) | 1280x1024 |
| Product labeling: |  |
| Manufacturer order number | 229200 |
| Series | eneo Monitor |
| Brand | eneo |

Additional Productinformation

Related products

Recommendations

| Jetrics jetcon HDMI Kabel 1 |  |
| --- | --- |
| Jetrics jetcon HDMI Kabel 3 |  |
| Jetrics jetcon HDMI Kabel 5 |  |
| Jetrics jetcon-BS-BS-1 | Videokabel mit 2 x BNC-Stecker 1m, 75 Ohm |
| Jetrics jetcon-BS-BS-2 | Videokabel mit 2 x BNC-Stecker 2m, 75 Ohm |
| Jetrics jetcon-BS-BS-3 | Videokabel mit 2 x BNC-Stecker 3m, 75 Ohm |
| Jetrics jetcon-BS-BS-5 | Videokabel mit 2 x BNC-Stecker 5m, 75 Ohm |

Services