Specifications text

Article number

233016

Type

MEB-62V2713P0A

Short description

1/2.8" HD Bullet, Day&Night, 1920x1080, Lens 2,7-13,5mm, IR LED, 12VDC, IP66

Main Features

1/2.8" 2.3 Megapixel progressive scan CMOS sensor

Compact housing for outdoor use

Resolution max. 1920x1080 (Full HD)

Multisignal: HD-TVI, AHD, CVI, Composite selectable

Variofocal 2.8~12mm day&night lens

LSC (Lens shading compensation)

Removable infrared cut filter

3D-DNR noise reduction

Digital Wide Dynamic Range (D-WDR)

Integral motion detection

4x High Power LED

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| Dimensions (HxWxD) | 78,5x78,5x214mm |
| Serial interfaces protocols | UTC (HD-Signal Format) |
| Resolution Pixels | 1920x1080, 960x576 |
| Resolution Pixels | 1920x1080, 960x576 |
| System | True Day&Night |
| Sensor size | 1/2.8" |
| Imager | CMOS |
| Day&Night switch | ICR |
| Sensitivity | 0,3 Lux at F1,6, 40 IRE |
| Digital Noise Reduction | yes |
| Lens Type | Varifocal |
| Focal Length | 2,7 - 13,5 mm |
| Iris Control | DC |
| Zoom Factor | 4x |
| Digital Zoom | no |
| Observe (62 px/m) | 16,6 - 80,5 m |
| Recognize (125 px/m) | 8,2 - 40,1 m |
| Identify (250 px/m) | 3,9 - 20,1 m |
| Detect (25px/m) | 44 - 198 m |
| Horizontal Angle of View | 95 - 27° |
| Vertical Angle of View | 49 - 15° |
| MOD (Mininum Object Distance) | 0,5 m |
| llumination | Infrared |
| Housing | Outdoor |
| Color (housing) | Pantone Cool Gray 1C |
| Hosuing material | Aluminium, Plastic |
| Video outputs | AHD, Composite (CVBS), CVI, HD-TVI |
| Alarm inputs | nein |
| Alarm outputs | no |
| Serial interfaces | Coaxitron (HD Protocol) |
| Analysis functions | Defog |
| Mounting | Ceiling mount, Wall mount |
| Supply voltage | 12VDC |
| Power consumption | 6.5W |
| Operating temperature | -20°C ~ +50°C |
| Protection rating | IP66 |
| Low Speed Shutter | yes |
| Backlight Compensation | BLC, HLC |
| Wide Dynamic Range (WDR) | D-WDR |
| Motion Detector | yes |
| Signal format | AHD @ 2MP, Composite (FBAS), CVI @ 2MP, HD-TVI @ 2MP |
| Camera type | Bullet |
| Frame rate | 25/30fps @ 1920x1080p |
| Aperture range (F) | F1.6 |
| Illumination range | approx. 30m |
| Weight | 520g |
| Manufacturer order number | 233016 |

Additional Productinformation

Related products

| AK-10 | Camera junction box for wall and ceiling mounting |
| --- | --- |
| MAM-5DC1004M0A | Distributor, 1x BNC Input HD-TVI, CVI, AHD, CVBS, 4x BNC Output |
| MAM-5MM1001M0A | Converter, HD-CVI, HD-TVI, AHD, Composite to HDMI, VGA, Composite, Indoor |
| TVA-1201TRA | UTP Transmitter+Receiver, UTP, BNC, 250m, Indoor, HD TVI CVI, AHD, CVBS, Active, 12V |
| TVA-1201TRP | UTP Transmitter+Receiver, UTP, BNC, 150m, Indoor, HD TVI CVI, AHD, CVBS, Passive |

Recommendations

| LACKIERUNG Bulletkamera | Extra charge for special paint bullet camera in RAL color |
| --- | --- |

Services

| LACKIERUNG Bulletkamera | Extra charge for special paint bullet camera in RAL color |
| --- | --- |
| LACKIERUNG Grundkosten | Painting basic costs once per RAL color and delivery lot |
| Vorkonfiguration analoge Kamera | Preconfiguration 1x analog camera Function test, FW update |