Specifications text

Article number

233016

Type

MEB-62V2713P0A

Short description

1/2.8" HD Bullet, Day&Night, 1920x1080, Lens 2,7-13,5mm, IR LED, 12VDC, IP66

Main Features

1/2.8" 2.3 Megapixel progressive scan CMOS sensor

Compact housing for outdoor use

Resolution max. 1920x1080 (Full HD)

Multisignal: HD-TVI, AHD, CVI, Composite selectable

Variofocal 2.8~12mm day&night lens

LSC (Lens shading compensation)

Removable infrared cut filter

3D-DNR noise reduction

Digital Wide Dynamic Range (D-WDR)

Integral motion detection

4x High Power LED

Other Features (Specifications)

| Audio support | No |
| --- | --- |
| Recording sensor | CMOS |
| Lighting | Infrared |
| Motion detector | No |
| Horizontal angle of view | 95 - 27° |
| Focal length | 2,7 - 13,5 mm |
| Chip size | 1/2,8" |
| Digital Noise Reduction (DNR) | Yes |
| Dimensions (WxHxD) (extended) | 78,5x78,5x214mm |
| Illumination range | ca. 30m |
| Remark | NDAA compliance under Section 889 |
| Watch (62px/m) | 166 m |
| Aperture Range (F) | F1.6 |
| External connections | video (BNC),12VDC power input,connection cable approx.40cm |
| Flickerless function | Yes |
| Identify (250px/m) | 39 m |
| Camera shape | Bullet |
| Scope of delivery | manual,drilling template,bag of screws |
| Menu languages | Chinese, Dutch, English, French, German, Italian, Polish, Portuguese, Russian, Spanish, Turkish |
| Multicast | No |
| Interface Protocols | UTC (HD Signal Format), UTC (HD Signal Format) |
| Signal-to-noise ratio | 52 dB |
| Signal Format | AHD @ 2MP, AHD @ 2MP, Composite (FBAS), CVI @ 2MP, HD-TVI @ 2MP, HD-TVI @ 2MP, HD-TVI @ 2MP, HD-TVI @ 2MP |
| Sunshade canopy | Yes |
| Text overlay | available |
| Perceiving (25px/m) | 44 m |
| Integrated wall mount | Yes |
| White balance | ATW,AWB,Manual,Outdoor,Indoor,AWC |
| Backlight compensation | No |
| Housing type | Outside |
| Housing material | Aluminium, Plastic |
| Heating | Yes |
| Photosensitivity | 0,3 Lux bei F1,6,40 IRE |
| Low Speed Shutter | No |
| Fan | No |
| Mounting type | Ceiling, Wall |
| Lens Type | Varifocal |
| Manufacturer order number | 233016 |
| Protection | IP66 |
| Control Interfaces | Coaxitron (HD Protocol) |
| Power supply | 12VDC |
| System | True Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| Temperature Range (Operating) | -20°C ~ +50°C |
| Vandalism-resistant | No |
| Wide Dynamic Range (WDR) | D-WDR, D-WDR, D-WDR, D-WDR, D-WDR, D-WDR, D-WDR |
| Zoomfactor (optical) | 4 |
| Brand | eneo |

Additional Productinformation

Related products

| AK-10 | Camera junction box for wall and ceiling mounting |
| --- | --- |
| MAM-5DC1004M0A | Distributor, 1x BNC Input HD-TVI, CVI, AHD, CVBS, 4x BNC Output |
| MAM-5MM1001M0A | Converter, HD-CVI, HD-TVI, AHD, Composite to HDMI, VGA, Composite, Indoor |
| TVA-1201TRA | UTP Transmitter+Receiver, UTP, BNC, 250m, Indoor, HD TVI CVI, AHD, CVBS, Active, 12V |
| TVA-1201TRP | UTP Transmitter+Receiver, UTP, BNC, 150m, Indoor, HD TVI CVI, AHD, CVBS, Passive |

Recommendations

| LACKIERUNG Bulletkamera | Extra charge for special paint bullet camera in RAL color |
| --- | --- |

Services

| LACKIERUNG Bulletkamera | Extra charge for special paint bullet camera in RAL color |
| --- | --- |
| LACKIERUNG Grundkosten | Painting basic costs once per RAL color and delivery lot |
| Vorkonfiguration analoge Kamera | Preconfiguration 1x analog camera Function test, FW update |