Specifications text

Article number

233052

Type

eneo MPC-52A0030M0A

Short description

1/3" HD Camera, 1920x1080, 30x AF Zoom 4,3-129mm, T-WDR, HD-SDI, EX-SDI, TVI, Composite

Main Features

1/3" 2,0 Megapixel Panasonic CMOS-Sensor

Resolution max. 1920x1080, 1080p@60/50fps

Optical 30x Auto Focus Zoom, 4,3~129mm

Multisignal Output HD-SDI, EX-SDI, TVI, Composite

Real time True WDR

Digital Noise Reduction(2D+3D)

Digital Image Stabilization

Privacy Masking, Defog

Slow Speed Shutter

Compact Housing

Power Supply 12VDC

Other Features (Specifications)

| General |  |
| --- | --- |
| Lighting | No |
| Dimensions (WxHxD) (extended) | 74x67x121,5mm |
| Weight | 0.61 kg |
| Text overlay | available |
| Color (Housing) | silvermetallic,black |
| Housing type | Inside |
| Housing material | Aluminium |
| Power consumption | 6 W |
| Power supply | 12VDC |
| Temperature Range (Operating) | -10°C ~ +50°C |
| Wide Dynamic Range (WDR) | WDR 120dB |
| Camera |  |
| Backlight compensation | Yes |
| Low Speed Shutter | Yes |
| Recording sensor | CMOS |
| Chip size | 1/3" |
| Flickerless function | Yes |
| Camera shape | Zoom cameras |
| Signal-to-noise ratio | 50 dB |
| Day/night mode | Automatic,Day,Night,extern (Sensor) |
| White balance | AUTO,One push,Manual,Indoor,Outdoor |
| Photosensitivity | 0,5 Lux bei F1,6,50 IRE |
| System | True Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| Display |  |
| Image resolution max. | 1920x1080,1280x720 |
| DORI |  |
| Watch (62px/m) | 27,8 - 800 m |
| Detect (125px/m) | 12,4 - 400 m |
| Identify (250px/m) | 5,6 - 190 m |
| Perceiving (25px/m) | 70,3 - 1800 m |
| Functions |  |
| Analysis | Motion Detection |
| Motion detector | Yes |
| Digital Noise Reduction (DNR) | Yes |
| Menu languages | Chinese, English, Japanese |
| Notes |  |
| Remark | NDAA compliance under Section 889 |
| Interfaces / Inputs/ Outputs |  |
| Number of alarm outputs | 1 |
| External connections | 1x SDI Signal (BNC),1x HD/SD Analog Signal (BNC),12VDC power input,RS-485,external day&night switching,Alarm output |
| Interface Protocols | Pelco D, Pelco P, UTC (HD Signal Format) |
| Control Interfaces | Coaxitron (HD Protocol), RS-485 |
| Video Outputs | Composite (FBAS) |
| Parts supplied |  |
| Scope of delivery | manual |
| Lens |  |
| Horizontal angle of view | 58,9 - 2,11° |
| Vertical angle of view | 45,3 - 1,61° |
| Focal length | 4,3 - 129 mm |
| Aperture Range (F) | F1.6~F4.7 |
| Lens Type | Auto Focus Zoom |
| Zoomfactor (optical) | 30 |
| Video |  |
| Signal Format | Composite (FBAS), EX-SDI @ 2MP, HD-SDI @ 2MP, HD-TVI @ 2MP |
| Horizontal resolution | 700 TVL |
| Product labeling: |  |
| Manufacturer order number | 233052 |
| Brand | eneo |

Additional Productinformation

Related products

Recommendations

Services