Specifications text

Article number

233051

Type

MPC-52A0010M0A

Short description

1/3" HD Camera, 1920x1080, 10x AF Zoom 5,1-51mm, T-WDR, HD-SDI, EX-SDI, TVI, Composite

Main Features

1/3" 2,0 Megapixel Panasonic CMOS-Sensor

Resolution max. 1920x1080, 1080p@60/50fps

Optical 10x Auto Focus Zoom, 5,1~51mm

Multisignal Output HD-SDI, EX-SDI, TVI, Composite

Real time True WDR

Digital Noise Reduction(2D+3D)

Digital Image Stabilization

Privacy Masking, Defog

Slow Speed Shutter

Compact Housing

Power Supply 12VDC

Other Features (Specifications)

| Number of alarm outputs | 1 |
| --- | --- |
| Audio support | No |
| Recording sensor | CMOS |
| Lighting | no |
| Motion detector | No |
| Horizontal angle of view | 54 - 4,9° |
| Focal length | 5,1 - 51 mm |
| Chip size | 1/3" |
| Digital Noise Reduction (DNR) | Yes |
| Dimensions (WxHxD) (extended) | 62x59x100mm |
| Watch (62px/m) | 33.2 m |
| Aperture Range (F) | F1.6~F1.8 |
| External connections | 1x SDI Signal (BNC),1x HD/SD Analog Signal (BNC),12VDC power input,RS-485,external day&night switching,Alarm output |
| Flickerless function | Yes |
| Identify (250px/m) | 8.1 m |
| Camera shape | Zoom cameras |
| Scope of delivery | manual |
| Menu languages | Chinese, English, Japanese |
| Multicast | No |
| Interface Protocols | Pelco D, Pelco P, UTC (HD Signal Format), UTC (HD Signal Format) |
| Signal-to-noise ratio | 50 dB |
| Signal Format | Composite (FBAS), EX-SDI @ 2MP, HD-SDI @ 2MP, HD-TVI @ 2MP, HD-TVI @ 2MP, HD-TVI @ 2MP, HD-TVI @ 2MP |
| Sunshade canopy | No |
| Day/night mode | Automatic,Day,Night,extern (Sensor) |
| Text overlay | available |
| Perceiving (25px/m) | 83.2 m |
| Integrated wall mount | No |
| White balance | AUTO,One push,Manual,Indoor,Outdoor |
| Backlight compensation | No |
| Housing type | Inside |
| Housing material | Aluminium |
| Heating | Yes |
| Horizontal resolution | 700 TVL |
| Photosensitivity | 0,5 Lux bei F1,6,50 IRE |
| Low Speed Shutter | No |
| Fan | No |
| Lens Type | Auto Focus Zoom |
| Manufacturer order number | 233051 |
| Control Interfaces | Coaxitron (HD Protocol), RS-485 |
| Power supply | 12VDC |
| System | True Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| Temperature Range (Operating) | -10°C ~ +50°C |
| Vandalism-resistant | No |
| Wide Dynamic Range (WDR) | WDR 120dB |
| Zoomfactor (optical) | 10 |
| Brand | eneo |

Additional Productinformation

Related products

Recommendations

Services