Specifications text

Article number

233631

Type

eneo VM-FHD22PP

Short description

22" (56cm) LCD Monitor, FHD, 1920x1080, LED, HDMI, VGA, RJ45 PoE Port

Main Features

Professional 22"(56 cm) LCD monitor

Maximum resolution Full HD 1920x1080 pixels

LED Back Light

3D filtering and noise reduction

Built to run 24x7x365 continuously

Different video inputs HDMI, DVI, VGA

Built-in speakers

Power supply by 12VDC or PoE input (RJ45)

Plastic cabinet including stand

Other Features (Specifications)

| General |  |
| --- | --- |
| Applications | Desktop, Industry |
| Dimensions (WxHxD) (extended) | 508,35x305,93x200mm |
| Fortification | VESA Standard 100x100mm |
| Weight | 3.5 kg |
| Energy efficiency class | E |
| Color (Housing) | Black |
| Housing material | Plastic |
| Power consumption | 25 W |
| Power supply | 12VDC |
| Display |  |
| Viewing Angle Horizontal | 178 ° |
| Viewing Angle Vertical | 178 ° |
| Screen diagonal (cm) | 56 cm |
| Screen diagonal (inches) | 22 " |
| Display | Anti-glare display, Color TFT-LCD, Hard coating (3H) |
| Pixel error class | ISO 13406-2, Class 2 |
| Brightness (typical) | 250 cd/m² |
| Backlight | LED |
| Contrast ratio (typical) | 1000:1 |
| Response time (typical) | 5 ms |
| Aspect ratio | 16:9 |
| Refresh rate | 60 Hz |
| Functions |  |
| Comb filter | Yes |
| Menu languages | English, French, German, Italian, Spanish |
| Notes |  |
| Remark | NDAA compliance under Section 889 |
| Interfaces / Inputs/ Outputs |  |
| Transmission Technology | HDCP 1.4, HDCP 2.2, HDMI 1.4a |
| Video Inputs | HDMI 2.0, VGA |
| Parts supplied |  |
| Scope of delivery | manual,PSU 100~240VAC,IR remote controller,feet |
| Video |  |
| Resolution (extended) | 800x600,1024x768,1280x1024,1600x900,1920x1080 |
| Maximum Image Resolution (HV) | 1920x1080 |
| Product labeling: |  |
| EAN/GTIN | 4260229841167 |
| Manufacturer order number | 233631 |
| Series | eneo Monitor |
| Brand | eneo |

Additional Productinformation

Related products

| eneo VM-LCDWMB2 | Wall bracket for LCD monitors horizontal adjustable, support up to max. 8 kg |
| --- | --- |
| eneo VM-PFF | Privacy filter foils for 22" LCD-Monitors |
| eneo VM-TSHA | Monitor stand high adjustable up to 130mm, for LCD-Monitors, 15-27 Inch, maximum 10Kg |
| Jetrics VM-UCWM20 | Compact slim tilt wall mount LCD Monitors, 23" to 42", VESA support up to 25Kg max. |
| Jetrics VM-UMA10 | Aluminium gas spring monitor arm for wall mount, LCD Monitors, max. 27" support up to 7 kg max. |
| Jetrics VM-UTM10 | Universal Table Mount for LCD Monitors, up to 45", VESA support up to 13Kg max. |
| Jetrics VM-UWM10 | Universal Wall Mount for LCD Monitors, up to max. 27", VESA support up to 15 kg max. |
| Jetrics VM-UWM30 | Universal Wall Mount for LCD Monitors, up to 43", VESA support up to 20Kg max. |

Recommendations

| Jetrics jetcon HDMI Kabel 1 |  |
| --- | --- |
| Jetrics jetcon HDMI Kabel 3 |  |
| Jetrics jetcon HDMI Kabel 5 |  |

Services