Specifications text

Article number

227047

Type

eneo ISM-78F0021W0A

Short description

1/2.8" Network Module Camera, 3840x2160, Day&Night, WDR, 2.1mm, H.265, PoE, 12VDC

Main Features

1/2.8" Sony STARVIS CMOS Sensor

Excellent light sensitivity

Resolution max. 3840x2160 (4K)

Lens fixed focal length 2.1mm

DOL-WDR (120dB) by triple scan images at 30/25fps

Motion Detection, Privacy Zones, Slow Shutter

Browser independent (IE, Edge, Safari, Firefox, Chrome)

Smart Codec H.265 ROI (Region of Interest)

Supports ONVIF Profile S and Profile T

Operating temperature -20°C ~ +50°C

Dimensions 50x50x64mm

Power supply PoE or 12VDC

Other Features (Specifications)

| General |  |
| --- | --- |
| Heating | No |
| Lighting | No |
| Dimensions (WxHxD) (extended) | 63x50x50mm |
| Weight | 0.112 kg |
| Text overlay | available |
| Housing type | without housing |
| Camera Type | Board Camera |
| Power consumption | 7 W |
| Mounting type | Built-in module |
| ONVIF | ONVIF Profile S, ONVIF Profile T |
| Power over Ethernet | IEEE 802.3af |
| Power supply | 12VDC, Poe |
| Temperature Range (Operating) | -20°C ~ +50°C |
| Wide Dynamic Range (WDR) | D-WDR |
| Camera |  |
| Backlight compensation | Yes |
| Low Speed Shutter | Yes |
| Resolution standard | 8 MP |
| Recording sensor | CMOS |
| Chip size | 1/2,8" |
| Flickerless function | Yes |
| Camera shape | Printed circuit board |
| Signal-to-noise ratio | 50 dB |
| Day/night mode | Automatic,Day,Night,extern (Sensor) |
| White balance | Automatic,Outdoor,Light bulb,Fluorescent light,Clear Sky,Shade,Manual |
| Photosensitivity | 0,03 Lux bei F2,0,40 IRE |
| System | True Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| Display |  |
| Image resolution max. | 3840x2160 |
| DORI |  |
| Watch (62px/m) | 28,5 m |
| Detect (125px/m) | 13,3 m |
| Identify (250px/m) | 5,2 m |
| Perceiving (25px/m) | 74,2 m |
| Functions |  |
| Analysis | Manipulation (TD), Motion Detection |
| Audio support | No |
| Motion detector | Yes |
| Digital Noise Reduction (DNR) | Yes |
| Multicast | Yes |
| Alarm processing | E-Mail,HTTP,FTP,Boost,TCP,UDP,Onvif Event Mapping |
| Browser | Edge: MJPEG, Mozilla: MJPEG, Safari: MJPEG |
| Menu languages | English, French, German, Korean |
| Password | Administrator and User |
| Notes |  |
| Remark | NDAA compliance under Section 889,To guarantee you an optimum user experience,we recommend that you only use components of the same brand in a system. If several brands are used in combination,eneo cannot guarantee functionality. If combined use is necess |
| Interfaces / Inputs/ Outputs |  |
| External connections | RJ-45,12VDC input (DC plug),connection cable approx.40cm |
| Control Interfaces | Ethernet |
| Parts supplied |  |
| Scope of delivery | manual |
| Lens |  |
| Horizontal angle of view | 109° |
| Vertical angle of view | 58° |
| Focal length | 2,1 mm |
| MOD (Minimum Object Distance) | 0.5 m |
| Lens Type | Prime lens |
| Video |  |
| Image transfer rate max. | 30 fps |
| Resolution (extended) | 3840x2160 (16:9),3072x1728 (16:9),2560x1440 (16:9),1920x1080 (16:9),1600x900 (16:9),1280x720 (16:9) |
| Data stream types | Multi-Streaming |
| Video Compression | H.264, H.264+, H.265, H.265+ |
| Product labeling: |  |
| Manufacturer order number | 227047 |
| Series | eneo AN-Serie |
| Brand | eneo |

Additional Productinformation

Related products

Recommendations

Services