Specifications text

Article number

232570

Type

eneo IPD-52M2713M0A

Short description

1/2.8" Network Dome, Fix, 1920x1080, Day/Night, DOL-WDR, 2.7-13.5mm, Infrared, IP66

Main Features

1/2.8" Sony STARVIS CMOS Sensor

Excellent light sensitivity

Resolution max. 1920x1080 (Full HD)

Motorised varifocal lens 2.7~13.5mm

DOL-WDR (120dB) by triple scan images at 30/25fps

Motion Detection, Privacy Zones, Slow Shutter

Browser independent (IE, Edge, Safari, Firefox, Chrome)

Smart Codec H.265 ROI (Region of Interest)

Supports ONVIF Profile S and Profile T

Vertical View 90°/270° Modus, H-Flip, Mirror

Fix Dome Housing (metal), IP66

Power supply PoE or 12VDC

Other Features (Specifications)

| General |  |
| --- | --- |
| Vandalism-resistant | Yes |
| Lighting | Infrared |
| Dimensions (WxHxD) (extended) | 144x109mm |
| Weight | 0.78 kg |
| SDK | upon request |
| Text overlay | available |
| Color (Housing) | Pantone Cool Gray 1C |
| Housing type | Outside |
| Housing material | Aluminium |
| Power consumption | 7.5 W |
| Mounting type | Ceiling, Wall |
| ONVIF | ONVIF Profile S, ONVIF Profile T |
| Power over Ethernet | IEEE 802.3af |
| Protection | IP66 |
| Power supply | 12VDC, Poe |
| Temperature Range (Operating) | -20°C ~ +50°C |
| Wide Dynamic Range (WDR) | D-WDR |
| Certifications | IK10 |
| Camera |  |
| Backlight compensation | Yes |
| Low Speed Shutter | Yes |
| Resolution standard | 2 MP |
| Recording sensor | CMOS |
| Chip size | 1/2,8" |
| Flickerless function | Yes |
| Camera shape | Minsters |
| Signal-to-noise ratio | 50 dB |
| Day/night mode | Automatic,Day,Night,extern (Sensor) |
| White balance | Automatic,Outdoor,Light bulb,Fluorescent light,Clear Sky,Shade,Manual |
| Photosensitivity | 0,0008 Lux bei F1,6,40 IRE |
| System | True Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| Display |  |
| Image resolution max. | 1920x1080 |
| DORI |  |
| Watch (62px/m) | 11,9 - 51,5 m |
| Detect (125px/m) | 5,6 - 25,4 m |
| Identify (250px/m) | 2,7 - 12,7 m |
| Perceiving (25px/m) | 30,6 - 127,5 m |
| Functions |  |
| Analysis | Manipulation (TD), Motion Detection |
| Audio support | No |
| Motion detector | Yes |
| Digital Noise Reduction (DNR) | Yes |
| Multicast | Yes |
| Alarm processing | E-Mail,HTTP,FTP,Boost,TCP,UDP,Onvif Event Mapping |
| Browser | Edge: MJPEG, Mozilla: MJPEG, Safari: MJPEG |
| Menu languages | English, French, German, Korean |
| Password | Administrator and User |
| Notes |  |
| Remark | NDAA compliance under Section 889,To guarantee you an optimum user experience,we recommend that you only use components of the same brand in a system. If several brands are used in combination,eneo cannot guarantee functionality. If combined use is necess |
| Interfaces / Inputs/ Outputs |  |
| External connections | 12VDC input (DC plug),RJ-45 |
| Control Interfaces | Ethernet |
| Parts supplied |  |
| Scope of delivery | manual,bag of screws |
| Lens |  |
| Horizontal angle of view | 108 - 32° |
| Vertical angle of view | 56 - 18° |
| Focal length | 2,7 - 13,5 mm |
| MOD (Minimum Object Distance) | 0.5 m |
| Lens Type | Varifocal (motorized) |
| Zoomfactor (optical) | 4 |
| Video |  |
| Image transfer rate max. | 30 fps |
| Resolution (extended) | 1920x1080 (16:9),1280x720 (16:9) |
| Data stream types | Multi-Streaming |
| Video Compression | H.264, H.264+, H.265, H.265+, M-JPEG |
| Product labeling: |  |
| Manufacturer order number | 232570 |
| Series | eneo AN-Serie |
| Brand | eneo |

Additional Productinformation

Related products

| eneo AK-11 | Camera junction box for wall and ceiling mounting |
| --- | --- |
| eneo DOME-WB3 | Wall bracket for eneo Dome Cameras MED-62xxxxxM0A and IED-62xxxxxM0A |

Recommendations

| Jetrics jetcon-Cat-Cat-FC | Feldkonfektionierbar |
| --- | --- |

Services

| PAINTING SERVICE basic costs | once per RAL color and delivery lot |
| --- | --- |
| PAINTING SERVICE fix dome camera | in RAL color |
| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |