Specifications text

Article number

232562

Type

ISM-62F0028W0A

Short description

1/2,8" Network Board Camera 1920x1080, Day/Night, WDR, 2,8mm, H.265, PoE, 12VDC

Main Features

1/2.8" Sony STARVIS CMOS Sensor

Excellent light sensitivity

Resolution max. 1920x1080 (Full HD)

Lens fixed focal length 2.8mm

WDR (120dB), BLC, 3DNR

Tripple Streaming (H.265, H.264, MJPEG)

Browser independent (IE, Edge, Safari, Firefox, Chrome)

Object classification (person, car, bike)

Supports ONVIF Profile S

Operating temperature -20°C +50°C

Dimensions 50x50x59mm

Power supply PoE or 12VDC

Other Features (Specifications)

| General |  |
| --- | --- |
| Lighting | no |
| Dimensions (WxHxD) (extended) | siehe Maßzeichnung |
| Text overlay | available |
| Housing type | without housing |
| Camera Type | Board Camera |
| Mounting type | Built-in module |
| ONVIF | ONVIF Profile S |
| Power over Ethernet | IEEE 802.3af |
| Power supply | 12VDC, Poe |
| Temperature Range (Operating) | -20°C ~ +50°C |
| Wide Dynamic Range (WDR) | WDR 120dB |
| Camera |  |
| Resolution standard | 2 MP |
| Recording sensor | CMOS |
| Chip size | 1/2,8" |
| Flickerless function | Yes |
| Camera shape | Printed circuit board |
| Signal-to-noise ratio | 50 dB |
| Day/night mode | Automatic,Day,Night,Time |
| White balance | ATW,Manual |
| Photosensitivity | 0,002 Lux bei F1,6 |
| System | True Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| Display |  |
| Image resolution max. | 1920x1080 |
| DORI |  |
| Watch (62px/m) | 17.7 m |
| Detect (125px/m) | 4.1 m |
| Identify (250px/m) | 4.1 m |
| Perceiving (25px/m) | 44.9 m |
| Functions |  |
| Audio support | Yes |
| Digital Noise Reduction (DNR) | Yes |
| Multicast | Yes |
| Alarm processing | alarm out,FTP,E-Mail |
| Audio | 2 Way Audio |
| Browser | Edge: MJPEG, Google Chrome: Java, Mozilla: MJPEG, MS Internet Explorer 11, Safari: MJPEG |
| Menu languages | English, German |
| Password | Administrator and User |
| Internal memory | no |
| Notes |  |
| Remark | NDAA compliance under Section 889,To guarantee you an optimum user experience,we recommend that you only use components of the same brand in a system. If several brands are used in combination,eneo cannot guarantee functionality. If combined use is necess |
| Interfaces / Inputs/ Outputs |  |
| Alarm inputs | 1 |
| External connections | 12VDC power input,Alarm in/output,Audio in/output,RJ-45 |
| Control Interfaces | Ethernet |
| Parts supplied |  |
| Scope of delivery | manual,Connection cable set |
| Lens |  |
| Horizontal angle of view | 113° |
| Focal length | 2,8 mm |
| MOD (Minimum Object Distance) | 0.5 m |
| Lens Type | Prime lens |
| Video |  |
| Image transfer rate max. | 30 fps |
| Resolution (extended) | 1920x1080 (16:9),1280x1024 (5:4),1024x768 (4:3),1280x720 (16:9),704x576 (4:3),704x480 (4:3),640x480 (4:3),640x360 (16:9),352x288 (4:3),352:240 (4:3) |
| Data stream types | Multi-Streaming |
| Video Compression | H.264, H.265 |
| Product labeling: |  |
| Manufacturer order number | 232562 |
| Brand | eneo |

Additional Productinformation

Related products

| eneo Site Manager | Management Software for eneo IP products |
| --- | --- |

Recommendations

Services

| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |
| --- | --- |