Specifications text

Article number

232564

Type

ISM-68F0028W0A

Short description

1/2,8" Netzwork Board Camera 3840x2160, Day/Night, WDR, 2,8mm, H.265, PoE, 12VDC

Main Features

1/2.8" Sony STARVIS CMOS Sensor

Excellent light sensitivity

Resolution max. 3840x2160 (4K)

Lens fixed focal length 2.8mm

WDR (120dB), BLC, 3DNR

Tripple Streaming (H.265, H.264, MJPEG)

Browser independent (IE, Edge, Safari, Firefox, Chrome)

Object classification (person, car, bike)

Supports ONVIF Profile S

Operating temperature -20°C +50°C

Dimensions 50x50x59mm

Power supply PoE or 12VDC

Other Features (Specifications)

| Alarm inputs | 1 |
| --- | --- |
| Audio support | Yes |
| Resolution standard | 8 MP |
| Recording sensor | CMOS |
| Lighting | no |
| Motion detector | No |
| Image resolution max. | 3840x2160 |
| Image transfer rate max. | 30 fps |
| Horizontal angle of view | 100° |
| Focal length | 2,8 mm |
| Chip size | 1/2,8" |
| Digital Noise Reduction (DNR) | Yes |
| Dimensions (WxHxD) (extended) | siehe Maßzeichnung |
| Alarm processing | alarm out,FTP,E-Mail |
| Audio | 2 Way Audio |
| Resolution (extended) | 3840x2160 (16:9),2592x1944 (4:3),2592x1520 (16:9),2304x1296 (16:9),2048x1536 (4:3),1920x1080 (16:9),1280x1024 (5:4),1024x768 (4:3),1280x720 (16:9),704x576 (4:3),704x480 (4:3),640x |
| Remark | NDAA compliance under Section 889,To guarantee you an optimum user experience,we recommend that you only use components of the same brand in a system. If several brands are used in combination,eneo cannot guarantee functionality. If combined use is necess |
| Watch (62px/m) | 36.8 m |
| Browser | Edge: MJPEG, Google Chrome: Java, Mozilla: MJPEG, MS Internet Explorer 11, Safari: MJPEG |
| Data stream types | Multi-Streaming |
| Detect (125px/m) | 8.1 m |
| External connections | 12VDC power input,Alarm in/output,Audio in/output,RJ-45 |
| Flickerless function | Yes |
| Identify (250px/m) | 8.1 m |
| Camera shape | Printed circuit board |
| Scope of delivery | manual,Connection cable set |
| Menu languages | English, German |
| Multicast | Yes |
| Password | Administrator and User |
| Signal-to-noise ratio | 50 dB |
| Sunshade canopy | No |
| Day/night mode | Automatic,Day,Night,Time |
| Text overlay | available |
| Perceiving (25px/m) | 90.7 m |
| Integrated wall mount | No |
| White balance | ATW,Manual |
| Backlight compensation | No |
| Housing type | without housing |
| Heating | Yes |
| Internal memory | no |
| Camera Type | Board Camera |
| Photosensitivity | 0,004 Lux bei F2,0 |
| Low Speed Shutter | No |
| MOD (Minimum Object Distance) | 0.5 m |
| Mounting type | Built-in module, Built-in module |
| Lens Type | Prime lens |
| ONVIF | ONVIF Profile S |
| Manufacturer order number | 232564 |
| Power over Ethernet | IEEE 802.3af |
| Control Interfaces | Ethernet |
| Power supply | 12VDC, Poe |
| System | True Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| Temperature Range (Operating) | -20°C ~ +50°C |
| Vandalism-resistant | No |
| Video Compression | H.264, H.265 |
| Wide Dynamic Range (WDR) | WDR 120dB |
| Wiper | No |
| Brand | eneo |

Additional Productinformation

Related products

Recommendations

Services