Specifications text

Article number

232563

Type

ISM-65F0028W0A

Short description

1/2,8" Network Board Camera 2592x1944, Day/Night, WDR, 2,8mm, H.265, PoE, 12VDC

Main Features

1/2.8" Sony STARVIS CMOS Sensor

Excellent light sensitivity

Resolution max. 2592x1944 (5MP)

Lens fixed focal length 2.8mm

WDR (120dB), BLC, 3DNR

Tripple Streaming (H.265, H.264, MJPEG)

Browser independent (IE, Edge, Safari, Firefox, Chrome)

Object classification (person, car, bike)

Supports ONVIF Profile S

Operating temperature -20°C +50°C

Dimensions 50x50x59mm

Power supply PoE or 12VDC

Other Features (Specifications)

| Audio support | Yes |
| --- | --- |
| Digital Noise Reduction (DNR) | Yes |
| Multicast | Yes |
| Heating | Yes |
| Alarm inputs | 1 |
| Resolution standard | 5 MP |
| Recording sensor | CMOS |
| Lighting | no |
| Image resolution max. | 2592x1944 |
| Image transfer rate max. | 30 fps |
| Horizontal angle of view | 105,2° |
| Focal length | 2,8 mm |
| Chip size | 1/2,8" |
| Dimensions (WxHxD) (extended) | siehe Maßzeichnung |
| Alarm processing | alarm out,FTP,E-Mail |
| Audio | 2 Way Audio |
| Resolution (extended) | 2592x1944 (4:3),2592x1520 (16:9),2304x1296 (16:9),2048x1536 (4:3),1920x1080 (16:9),1280x1024 (5:4),1024x768 (4:3),1280x720 (16:9),704x576 (4:3),704x480 (4:3),640x480 (4:3),640x360 |
| Remark | NDAA compliance under Section 889,To guarantee you an optimum user experience,we recommend that you only use components of the same brand in a system. If several brands are used in combination,eneo cannot guarantee functionality. If combined use is necess |
| Watch (62px/m) | 26.1 m |
| Browser | Edge: MJPEG, Google Chrome: Java, Mozilla: MJPEG, MS Internet Explorer 11, Safari: MJPEG |
| Data stream types | Multi-Streaming |
| Detect (125px/m) | 5.7 m |
| External connections | 12VDC power input,Alarm in/output,Audio in/output,RJ-45 |
| Flickerless function | Yes |
| Identify (250px/m) | 5.7 m |
| Camera shape | Printed circuit board |
| Scope of delivery | manual,Connection cable set |
| Menu languages | English, German |
| Password | Administrator and User |
| Signal-to-noise ratio | 50 dB |
| Day/night mode | Automatic,Day,Night,Time |
| Text overlay | available |
| Perceiving (25px/m) | 66.7 m |
| White balance | ATW,Manual |
| Housing type | without housing |
| Internal memory | no |
| Camera Type | Board Camera |
| Photosensitivity | 0,003 Lux bei F1,6 |
| MOD (Minimum Object Distance) | 0.5 m |
| Mounting type | Built-in module |
| Lens Type | Prime lens |
| ONVIF | ONVIF Profile S |
| Manufacturer order number | 232563 |
| Power over Ethernet | IEEE 802.3af |
| Control Interfaces | Ethernet |
| Power supply | 12VDC, Poe |
| System | True Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| Temperature Range (Operating) | -20°C ~ +50°C |
| Video Compression | H.264, H.265 |
| Wide Dynamic Range (WDR) | WDR 120dB |
| Brand | eneo |

Additional Productinformation

Related products

| eneo Site Manager | Management Software for eneo IP products |
| --- | --- |

Recommendations

Services

| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |
| --- | --- |