Specifications text

Article number

230801

Type

ICB-62M2713MAA

Short description

1/2.8" Network Camera, 1920x1080, Day/Night, DOL-WDR, 2.7-13.5mm, Infrared, IP67

Main Features

1/2.8" Sony STARVIS CMOS Sensor

Excellent light sensitivity

Resolution max. 1920x1080 (Full HD)

Motorised varifocal lens 2.7~13.5mm

DOL-WDR (120dB) by triple scan images at 30/25fps

Motion Detection, Privacy Zones, Slow Shutter

Browser independent (IE, Edge, Safari, Firefox, Chrome)

Smart Codec H.265 ROI (Region of Interest)

Supports ONVIF Profile S and Profile T

Vertical View 90°/270° Modus, H-Flip, Mirror

Power supply PoE, IP67

Integrated connection box including connection board

Other Features (Specifications)

| Alarm inputs | 1 |
| --- | --- |
| Audio support | No |
| Resolution standard | 0 MP |
| Recording sensor | CMOS |
| Lighting | Infrared |
| Motion detector | No |
| Image resolution max. | 1920x1080 |
| Image transfer rate max. | 30 fps |
| Horizontal angle of view | 108 - 32° |
| Focal length | 2,7 - 13,5 mm |
| Chip size | 1/2,8" |
| Digital Noise Reduction (DNR) | Yes |
| Dimensions (WxHxD) (extended) | 130,50x82,15x265,80mm |
| Alarm processing | recording internal storage (SD card),E-Mail,HTTP,FTP,Boost,TCP,UDP,Onvif Event Mapping,alarm out |
| Resolution (extended) | 1920x1080 (16:9),1280x720 (16:9) |
| Remark | NDAA compliance under Section 889,To guarantee you an optimum user experience,we recommend that you only use components of the same brand in a system. If several brands are used in combination,eneo cannot guarantee functionality. If combined use is necess |
| Watch (62px/m) | 166 m |
| Browser | Edge: MJPEG, Mozilla: MJPEG, Safari: MJPEG |
| Data stream types | Multi-Streaming |
| Detect (125px/m) | 39 m |
| External connections | micro SD card slot,Connection board (LSA,RJ45,audio,I / O alarm & Sabotage contacts) |
| Flickerless function | Yes |
| Identify (250px/m) | 39 m |
| Camera shape | Bullet |
| Scope of delivery | manual,bag of screws |
| Menu languages | English, French, German, Korean |
| Multicast | Yes |
| Password | Administrator and User |
| SDK | upon request |
| Signal-to-noise ratio | 50 dB |
| Sunshade canopy | Yes |
| Day/night mode | Automatic,Day,Night,extern (Sensor) |
| Text overlay | available |
| Perceiving (25px/m) | 44 m |
| Integrated wall mount | Yes |
| White balance | Automatic,Outdoor,Light bulb,Fluorescent light,Clear Sky,Shade,Manual |
| Backlight compensation | No |
| Housing type | Outside |
| Housing material | Aluminium |
| Heating | Yes |
| Internal memory | microSDHC card, microSDXC card |
| Photosensitivity | 0,0008 Lux bei F1,6,40 IRE |
| Low Speed Shutter | No |
| MOD (Minimum Object Distance) | 0.5 m |
| Mounting type | Ceiling, Wall |
| Lens Type | Varifocal (motorized) |
| ONVIF | ONVIF Profile S, ONVIF Profile T |
| Manufacturer order number | 230801 |
| Power over Ethernet | IEEE 802.3af |
| Protection | IP67 |
| Series | eneo Candid Small,eneo AN-Serie |
| Control Interfaces | Ethernet |
| Power supply | Poe |
| System | True Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| Temperature Range (Operating) | -50°C ~ +50°C |
| Vandalism-resistant | No |
| Video Compression | H.264, H.264+, H.265, H.265+, M-JPEG |
| Wide Dynamic Range (WDR) | D-WDR, D-WDR, D-WDR, D-WDR, D-WDR, D-WDR, D-WDR |
| Wiper | No |
| Zoomfactor (optical) | 4 |
| Brand | eneo |

Additional Productinformation

Related products

Recommendations

Services