Specifications text

Article number

233015

Type

eneo MED-62V2713M0A

Short description

HD Analogue Dome, Fix, Day&Night 1920x1080, Lens 2,7-13,5mm, IR LED, 12VDC, IP66, IK10

Main Features

1/2.8" 2.3 Megapixel progressive scan CMOS sensor

Compact metal housing for outdoor use

Resolution max. 1920x1080 (Full HD)

Multisignal: HD-TVI, AHD, CVI, Composite selectable

Variofocal 2.8~12mm day&night lens

LSC (Lens shading compensation)

Removable infrared cut filter

3D-DNR noise reduction

Integral motion detection

Scratch resistant hard coated bubble

Easy installation, 3-Axis gimbal

Other Features (Specifications)

| General |  |
| --- | --- |
| Vandalism-resistant | Yes |
| Lighting | Infrared |
| Dimensions (WxHxD) (extended) | 108x144,4mm |
| Illumination range | ca. 30m |
| Weight | 1 kg |
| Text overlay | Switchable on/off |
| Color (Housing) | Pantone Cool Gray 1C |
| Color (dome) | clear |
| Housing type | Outside |
| Housing material | Aluminium |
| Power consumption | 6 W |
| Mounting type | Ceiling, Wall |
| Protection | IP66 |
| Power supply | 12VDC |
| Temperature Range (Operating) | -20°C ~ +50°C |
| Wide Dynamic Range (WDR) | D-WDR |
| Certifications | IK10 |
| Camera |  |
| Backlight compensation | Yes |
| Low Speed Shutter | Yes |
| Recording sensor | CMOS, Sony IMX323 |
| Chip size | 1/2,8" |
| Automatic Electronic Shutter (AES) | 1/25 ~ 1/50,000sec |
| Flickerless function | Yes |
| Camera shape | Minsters |
| Manual Electric Shutter (MES) | 1-25~1-50,000sec |
| Signal-to-noise ratio | 52 dB |
| Day/night mode | Yes |
| White balance | ATW,AWB,Manual,Outdoor,Indoor,AWC |
| Photosensitivity | 0,3 Lux bei F1,6,40 IRE |
| System | True Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| Display |  |
| Image resolution max. | 1920x1080 |
| DORI |  |
| Watch (62px/m) | 16,6 - 80,5 m |
| Detect (125px/m) | 8,2 - 40,1 m |
| Identify (250px/m) | 3,9 - 20,1 m |
| Perceiving (25px/m) | 44 - 198 m |
| Functions |  |
| Motion detector | Yes |
| Digital Noise Reduction (DNR) | Yes |
| Menu languages | Chinese, Dutch, English, French, German, Italian, Polish, Portuguese, Russian, Spanish, Turkish |
| Privacy Zone | Areas-ON/OFF switchable |
| Vertical tilt range | 65 °(25 ~ 90°) |
| Notes |  |
| Remark | NDAA compliance under Section 889 |
| Interfaces / Inputs/ Outputs |  |
| External connections | video (BNC),12VDC power input,connection cable approx.40cm |
| Interface Protocols | UTC (HD Signal Format) |
| Control Interfaces | Coaxitron (HD Protocol) |
| Video Outputs | AHD, Composite (FBAS), CVI |
| Parts supplied |  |
| Scope of delivery | Quick installation guide,drilling template,4 wall plugs with screws,1 Allen wrench,video sub-output cable |
| Lens |  |
| Horizontal angle of view | 95 - 27° |
| Vertical angle of view | 49 - 15° |
| Focal length | 2,7 - 13,5 mm |
| Aperture Range (F) | F1.6 |
| Lens Type | Varifocal |
| Zoomfactor (optical) | 4 |
| Video |  |
| Resolution (extended) | 1920x1080,960x576 |
| Signal Format | AHD @ 2MP, Composite (FBAS), CVI @ 2MP, HD-TVI @ 2MP |
| Product labeling: |  |
| EAN/GTIN | 4260229841433 |
| Manufacturer order number | 233015 |
| Series | eneo COAXIZE |
| Brand | eneo |

Additional Productinformation

Related products

| eneo AK-11 | Camera junction box for wall and ceiling mounting |
| --- | --- |
| eneo DOME-WB3 | Wall bracket for eneo Dome Cameras MED-62xxxxxM0A and IED-62xxxxxM0A |

Recommendations

| eneo MAM-5DC1004M0A | Distributor, 1x BNC Input HD-TVI, CVI, AHD, CVBS, 4x BNC Output |
| --- | --- |
| eneo MAM-5MM1001M0A | Converter, HD-CVI, HD-TVI, AHD, Composite to HDMI, VGA, Composite, Indoor |
| eneo TVA-1201TRA | UTP Transmitter+Receiver, UTP, BNC, 250m, Indoor, HD TVI CVI, AHD, CVBS, Active, 12V |
| eneo TVA-1201TRP | UTP Transmitter+Receiver, UTP, BNC, 150m, Indoor, HD TVI CVI, AHD, CVBS, Passive |
| Jetrics jetcon-BS-BS-1 | Videokabel mit 2 x BNC-Stecker 1m, 75 Ohm |
| Jetrics jetcon-BS-BS-2 | Videokabel mit 2 x BNC-Stecker 2m, 75 Ohm |
| Jetrics jetcon-BS-BS-3 | Videokabel mit 2 x BNC-Stecker 3m, 75 Ohm |
| Jetrics jetcon-BS-BS-5 | Videokabel mit 2 x BNC-Stecker 5m, 75 Ohm |
| Jetrics jetpower-PSU-XS-11C Steckernetzteil |  |

Services

| PAINTING SERVICE basic costs | once per RAL color and delivery lot |
| --- | --- |
| PAINTING SERVICE fix dome camera | in RAL color |
| Vorkonfiguration analoge Kamera | Preconfiguration 1x analog camera Function test, FW update |