Specifications text

Article number

233036

Type

VM-HD19MPA

Short description

19" (48cm) LCD, Industrial Monitor, 1280x1024, HDMI, VGA, Composite, Protection Glass

Main Features

Professional 19" (48 cm) LCD monitor

Built to run 24x7x365 continuously (Industrial Level)

Including protection glass, Anti-Glare-Display 3H

Maximum resolution 1280x1024 pixels

LED Back Light

Fast response time 5ms

Different video inputs HDMI, VGA, BNC (CVBS)

Audio In- and Output

Built-in speakers

Metal cabinet including wall mount

Other Features (Specifications)

| Audio Inputs | 1 |
| --- | --- |
| Viewing Angle Horizontal | 178 ° |
| Viewing Angle Vertical | 178 ° |
| Screen diagonal (cm) | 48 cm |
| Screen diagonal (inches) | 19 " |
| Display | Anti-glare display, Glass, Hard coating (6.5H), Haze: 89%, Not anit-glare type |
| Scope of delivery | manual,power cord,wall bracket,HDMI cable |
| Menu languages | Chinese, English, French, German, Italian |
| Transmission Technology | HDCP 1.4, HDMI 1.4a |
| Housing material | Metal |
| Brightness (typical) | 250 cd/m² |
| Backlight | LED |
| Contrast ratio (typical) | 1000:1 |
| Manufacturer order number | 233036 |
| Response time (typical) | 14 ms |
| Aspect ratio | 5:4 |
| Series | eneo Monitor |
| Control Interfaces | no |
| Video Inputs | Composite (FBAS), HDMI, VGA |
| Brand | eneo |
| Refresh rate | 60 Hz |

Additional Productinformation

Related products

| VM-LCDWMB2 | Wall bracket for LCD monitors horizontal adjustable, support up to max. 8 kg |
| --- | --- |
| VM-TSHA | Monitor stand high adjustable up to 130mm, for LCD-Monitors, 15-27 Inch, maximum 10Kg |
| VM-UCWM20 | Compact slim tilt wall mount LCD Monitors, 23" to 42", VESA support up to 25Kg max. |
| VM-UMA10 | Aluminium gas spring monitor arm for wall mount, LCD Monitors, max. 27" support up to 7 kg max. |
| VM-UTM10 | Universal Table Mount for LCD Monitors, up to 45", VESA support up to 13Kg max. |
| VM-UWM10 | Universal Wall Mount for LCD Monitors, up to max. 27", VESA support up to 15 kg max. |
| VM-UWM30 | Universal Wall Mount for LCD Monitors, up to 43", VESA support up to 20Kg max. |

Recommendations

| jetcon HDMI Kabel 1 |  |
| --- | --- |
| jetcon HDMI Kabel 10 |  |
| jetcon HDMI Kabel 3 |  |

Services