Specifications text

Article number

233902

Type

IND-45F0028M0A

Short description

Mini Network Dome Fix, 2880x1620, Day/Night, WDR, AI, 2,8mm, IR, PoE, IP66

Main Features

1/2.7" CMOS sensor, maximum resolution 5 MP/30fps

AI analysis functions including object classification

WDR 120dB, 3D-DNR noise reduction

Fixed focal length lens 2.8mm (HFoV 110°)

Smart Codec H.265/H.264, multi-streaming

Browser independent (IE, Edge, Safari, Firefox, Chrome)

Supports ONVIF Profile S and Profile T

Multilingual menu navigation (OSD)

Local storage via Micro SD card

Supports audio (built-in microphone)

Power supply PoE IEEE 802.3af

IR LED illumination, IP66

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| EAN/GTIN | 4260229840009 |
| Dimensions (HxWxD) | 115,3x64,3mm |
| Series | eneo IN-Series |
| Camera Type | Face recognition, License plate recognition, People Count, Retail Intelligence |
| Resolution Standard | 5 Megapixel |
| System | True Day&Night |
| Sensor size | 1/2.7" |
| Imager | CMOS |
| Video compression | H.264, H.264+ (Smart Stream), H.265, H.265+ (Smart Stream), M-JPEG |
| Image resolution max. | 2880x1620 |
| Image Transmission Rate | 30 fps |
| Day&Night switch | ICR |
| Sensitivity | 0,004 Lux at F1,6 |
| Digital Noise Reduction | yes |
| Lens Type | fixed focal length |
| Focal Length | 2,8 mm |
| Iris Control | DC |
| Digital Zoom | yes |
| Observe (62 px/m) | 25,4 m |
| Recognize (125 px/m) | 12,1 m |
| Identify (250 px/m) | 5,4 m |
| Detect (25px/m) | 64,4 m |
| Horizontal Angle of View | 110° |
| Vertical Angle of View | 58° |
| MOD (Mininum Object Distance) | 1,8 m |
| llumination | Infrared |
| Housing | Outdoor |
| Color (housing) | Pantone P 179-1, RAL9003 |
| Color (bubble) | clear |
| Hosuing material | Aluminium, Plastic |
| Heater | no |
| Blower | no |
| Alarm inputs | nein |
| Alarm outputs | no |
| Audio support | yes |
| Ethernet interface | 100Base-TX, 10Base-T |
| Serial interfaces | Ethernet |
| Internal storage | microSD-Kartenslot |
| Analysis functions | Attribute Detection (AD), Cross Counting (CC), Crowd Density Detection (CD), Face Attendance (FA), Face Detection (FD), Face Recogntion (FR), Heat Map (HM), Human&Vehicle Detection (PD&VD), License Plate Detection (LPD), License Plate Recognition (LPR), Line Crossing Detection (LCD), Perimeter Intrusion Detection (PID), Queue Lenght Detection (QD), Rare Sound Detection (RSD), Stationary Object Detection (SOD) |
| Mounting | Ceiling mount, Wall mount |
| Supply voltage | PoE |
| Power consumption | 4.5W |
| Power over Ethernet | IEEE 802.3af |
| Vandal proof | no |
| Operating temperature | -35°C ~ +60°C, @ Humidity 90% or less (non-condensing) |
| ONVIF | ONVIF Profile G, ONVIF Profile S, ONVIF Profile T |
| Protection rating | IP66 |
| Low Speed Shutter | yes |
| Backlight Compensation | BLC, HLC |
| Wide Dynamic Range (WDR) | WDR 120dB |
| Motion Detector | yes |
| Camera type | Dome |
| Audio | 1 Way Audio, built-in microphone |
| Remote configuration | Browser |
| Illumination range | approx. 20m |
| Weight | 265g |
| Manufacturer order number | 233902 |

Additional Productinformation

Related products

| eneo INsight Installation Tool | Supports adjustments for cameras with AI features for IN-series |
| --- | --- |
| eneo INsight mobile | Mobile App for recorder IN- and MN-series |
| eneo Site Manager | Management Software for eneo IP products |

Recommendations

| eneo INsight mobile | Mobile App for recorder IN- and MN-series |
| --- | --- |
| jetcon-Cat-Cat-FC | Kabelverbinder AWG27-22, IP67 Feldkonfektionierbar |
| jetcon-RJ45S-Cat6-FC | RJ45 Stecker Cat.6 AWG22-27, Feldkonfektionierbar, schwarz |
| jetcon-RJ45S-Cat6-small-FC | RJ45 Stecker Cat.6 AWG22-26, Feldkonfektionierbar, schmale und kurze Ausführung |
| jetcon-RJ45S-Cat6A-FC | RJ45 Stecker Cat.6A AWG22-27 Feldkonfektionierbar, blau |
| LACKIERUNG Fix-Dome-Kamera | Extra charge for special paint fixed dome in RAL color |

Services

| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |
| --- | --- |