Specifications text

Article number

234751

Type

INP-75A0033MIA

Short description

Netzwerk Dome, PTZ, 33x 2952x1944, Tag/Nacht, WDR, AI Alarm, Audio, 24/12V, PoE,IP66

Main Features

1/2.7" CMOS sensor, maximum resolution 5 MP/30fps

AI analysis functions including object classification

WDR 120dB, 3D-DNR Noise Reduction

Motorized autofocus zoom lens 33x 4.3-142mm

Smart Codec H.265/H.264, multi-streaming

Browser independent (IE, Edge, Safari, Firefox, Chrome)

Supports ONVIF Profile S and Profile T

Local storage via Micro SD card

Audio input and output

IR LED illumination, IP66

NDAA compliance

Wall bracket included

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| EAN/GTIN | 4260229840160 |
| Dimensions (HxWxD) | 206,4×351,8mm |
| Series | eneo IN-Series |
| Camera Type | Face recognition, People Count, Retail Intelligence |
| Resolution Standard | 5 Megapixel |
| System | True Day&Night |
| Sensor size | 1/2.7" |
| Imager | CMOS |
| Video compression | H.264, H.264+ (Smart Stream), H.265, H.265+ (Smart Stream), M-JPEG |
| Image resolution max. | 2592x1944 |
| Image Transmission Rate | 30 fps |
| Day&Night switch | ICR |
| Sensitivity | 0,003 Lux at F1,6 |
| Digital Noise Reduction | yes |
| Lens Type | auto focus zoom |
| Focal Length | 4,3 - 142 mm |
| Iris Control | DC |
| Zoom Factor | 33x |
| Observe (62 px/m) | 35,6 - 1107,3 m |
| Recognize (125 px/m) | 17 - 548,3 m |
| Identify (250 px/m) | 7,5 - 273,3 m |
| Detect (25px/m) | 90,2 - 2728,4 m |
| Horizontal Angle of View | 55 - 2,3° |
| Vertical Angle of View | 41,6 - 1,7° |
| llumination | Infrared |
| Housing | Outdoor |
| Color (housing) | black, Pantone P 179-1, RAL9003 |
| Hosuing material | Aluminium |
| Heater | no |
| Blower | no |
| Rotation range | 360° |
| Speed (manual) | 200°/Sek. |
| Speed (preset) | 200°/Sek. |
| Alarm inputs | nein |
| Alarm outputs | no |
| Audio support | yes |
| Ethernet interface | 100Base-TX, 10Base-T |
| Serial interfaces | Ethernet |
| Internal storage | microSD-Kartenslot |
| Analysis functions | Cross Counting (CC), Face Detection (FD), Face Recogntion (FR), Heat Map (HM), Human&Vehicle Detection (PD&VD), Line Crossing Detection (LCD), Perimeter Intrusion Detection (PID), Stationary Object Detection (SOD), Tampering Detection (TD) |
| Mounting | Wall mount |
| Supply voltage | 12VDC, PoE+ |
| Power consumption | 25,5W |
| Power over Ethernet | IEEE 802.3at |
| Vandal proof | no |
| Operating temperature | -30°C ~ +60°C, @ Humidity 90% or less (non-condensing) |
| ONVIF | ONVIF Profile G, ONVIF Profile S, ONVIF Profile T |
| Protection rating | IP66 |
| Low Speed Shutter | yes |
| Backlight Compensation | BLC, HLC |
| Wide Dynamic Range (WDR) | WDR 120dB |
| Motion Detector | yes |
| Camera type | PTZ |
| Audio | 1/1 Line input and output, 2 Way Audio |
| Remote configuration | Browser |
| Illumination range | approx. 150m |
| Weight | 3.7kg |
| Manufacturer order number | 234751 |

Additional Productinformation

Related products

| CM-22 | Corner mounting adapter for wall bracket WB-24 and PTZ dome camera INP-75A0033MIA |
| --- | --- |
| eneo INsight Installation Tool | Supports adjustments for cameras with AI features for IN-series |
| eneo INsight mobile | Mobile App for recorder IN- and MN-series |
| eneo Site Manager | Management Software for eneo IP products |
| KBD-3NET | Network keyboard with joystick, supports ONVIF protocol, RJ45, 12VDC |
| PM-21 | Pole mounting adapter for camera connection box AK-20 and AK-21 |

Recommendations

| CM-22 | Corner mounting adapter for wall bracket WB-24 and PTZ dome camera INP-75A0033MIA |
| --- | --- |
| eneo INsight mobile | Mobile App for recorder IN- and MN-series |
| jetpower-PSU-S | Universal Netzteil 12V/5A ID 2.1 x OD 5.5 |
| PM-21 | Pole mounting adapter for camera connection box AK-20 and AK-21 |

Services

| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |
| --- | --- |