Specifications text

Article number

234751

Type

eneo INP-75A0033MIA

Short description

Network Dome, PTZ, 33x, 2952x1944, Day & Night, WDR, AI, Alarm, Audio, 24VAC/12VDC, PoE, IP66

Main Features

1/2.7" CMOS sensor, maximum resolution 5 MP/30fps

AI analysis functions including object classification

WDR 120dB, 3D-DNR Noise Reduction

Motorized autofocus zoom lens 33x 4.3-142mm

Smart Codec H.265/H.264, multi-streaming

Browser independent (IE, Edge, Safari, Firefox, Chrome)

Supports ONVIF Profile S and Profile T

Local storage via Micro SD card

Audio input and output

IR LED illumination, IP66

Wall bracket included

Other Features (Specifications)

| General |  |
| --- | --- |
| Heating | No |
| Lighting | Infrared |
| Dimensions (WxHxD) (extended) | 206,4×351,8mm |
| Weight | 3.7 kg |
| Color (Housing) | RAL9003,Pantone P 179-1,black |
| Housing type | Outside |
| Housing material | Aluminium |
| Camera Type | Face, People counting, Retail Intelligence |
| Power consumption | 25.5 W |
| Mounting type | Wall |
| ONVIF | ONVIF Profile G, ONVIF Profile S, ONVIF Profile T |
| Power over Ethernet | IEEE 802.3at |
| Protection | IP66 |
| Power supply | 12VDC, PoE+ |
| Temperature Range (Operating) | -30°C ~ +60°C,@ Luftfeuchtigkeit 90% oder weniger (nicht kondensierend) |
| Wide Dynamic Range (WDR) | WDR 120dB |
| Camera |  |
| Backlight compensation | Yes |
| Low Speed Shutter | Yes |
| Resolution standard | 5 MP |
| Recording sensor | CMOS |
| Chip size | 1/2,7" |
| Flickerless function | Yes |
| Camera shape | PTZ |
| Day/night mode | Automatic,Day,Night,Image,Time |
| White balance | Automatic,Manual |
| Photosensitivity | 0,003 Lux bei F1,6 |
| System | True Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| Display |  |
| Image resolution max. | 2592x1944 |
| DORI |  |
| Watch (62px/m) | 42,5 - 1041 m |
| Detect (125px/m) | 20,5 - 515 m |
| Identify (250px/m) | 10 - 258 m |
| Perceiving (25px/m) | 106 - 2596 m |
| Functions |  |
| Analysis | Abandoned and Removed Objects (SOD), Face Capture (FD), Face Recognition (FR), Heat Map (HM), Intrusion (PID), Line crossing (LCD), Manipulation (TD), Passage count (CC) |
| Audio support | Yes |
| Motion detector | Yes |
| Digital Noise Reduction (DNR) | Yes |
| Multicast | Yes |
| Turning range | 360 |
| Alarm processing | recording internal storage (SD card),E-Mail,FTP,Cloud Upload |
| Audio | 2 Way Audio |
| Browser | Edge, Firefox, Google Chrome, Safari |
| Menu languages | English, French, German, Italian, Japanese, Portuguese, Russian, Spanish, Turkish |
| Password | Administrator and User |
| Tours | 1 selectable tour |
| Speed (manual control) | 200 °/sec |
| Speed (Target Approach) | 200 °/sec |
| Internal memory | microSD card slot |
| Notes |  |
| Remark | The face recognition functions are only available with an inserted SD card or in conjunction with a recorder of the eneo IN series.,To guarantee you an optimum user experience,we recommend that you only use components of the same brand in a system. If sev |
| Interfaces / Inputs/ Outputs |  |
| External connections | RJ-45,Audio input/output,12VDC input (DC plug),connection cable approx.45cm,micro SD card slot 256 GB |
| Control Interfaces | Ethernet |
| Parts supplied |  |
| Scope of delivery | manual,wall bracket,bag of screws |
| Lens |  |
| Horizontal angle of view | 55 - 2,3° |
| Vertical angle of view | 41,6 - 1,7° |
| Focal length | 4,3 - 142 mm |
| MOD (Minimum Object Distance) | 0 m |
| Lens Type | Auto Focus Zoom |
| Zoomfactor (optical) | 33 |
| Video |  |
| Image transfer rate max. | 30 fps |
| Resolution (extended) | 2592x1944 (4:3),2592x1520 (16:9),2304x1296 (16:9),1920x1080 (16:9),1280x960 (4:3),640x480 (4:3),320x240 (4:3) |
| Data stream types | Triple-Stream(Main/Sub/Mobile) |
| Video Compression | H.264, H.264+, H.265, H.265+, M-JPEG |
| Product labeling: |  |
| EAN/GTIN | 4260229840160 |
| Manufacturer order number | 234751 |
| Series | eneo IN-Serie |
| Brand | eneo |

Additional Productinformation

Related products

Recommendations

Services