Specifications text

Article number

234752

Type

INP-58M2812M0A

Short description

Network Dome, PTZ, 4x MVF, 3840x2160, Day/Night, WDR, AI, Alarm, Audio, 12V, PoE, IP66

Main Features

1/2.7" CMOS sensor, maximum resolution 8 MP/30fps

AI analysis functions including object classification

D-WDR, 3D-DNR noise reduction

Motorized varifocal lens 2.8-12mm

Smart Codec H.265/H.264, multi-streaming

Browser independent (IE, Edge, Safari, Firefox, Chrome)

Supports ONVIF Profile S and Profile T

Local storage via Micro SD card

Supports audio and external alarm input and output

IR LED lighting, IP66, deterrent (light & siren)

NDAA compliance

Wall and ceiling mounting included

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| EAN/GTIN | 4260229840115 |
| Dimensions (HxWxD) | 166,75x238mm |
| Series | eneo IN-Series |
| Camera Type | Face recognition, License plate recognition, People Count, Retail Intelligence |
| Resolution Standard | 8 Megapixel |
| System | True Day&Night |
| Sensor size | 1/2.7" |
| Imager | CMOS |
| Video compression | H.264, H.264+ (Smart Stream), H.265, H.265+ (Smart Stream), M-JPEG |
| Image resolution max. | 3840x2160 |
| Image Transmission Rate | 25 fps |
| Day&Night switch | ICR |
| Sensitivity | 0,009 Lux at F1,6 |
| Digital Noise Reduction | yes |
| Lens Type | Varifocal (motorized) |
| Focal Length | 2,8 - 12 mm |
| Iris Control | DC |
| Digital Zoom | yes |
| Observe (62 px/m) | 35,6 - 139,4 m |
| Recognize (125 px/m) | 17 - 68,6 m |
| Identify (250 px/m) | 8,2 - 35,2 m |
| Detect (25px/m) | 90 - 346,2 m |
| Horizontal Angle of View | 113 - 31,4° |
| Vertical Angle of View | 58,4 - 17,6° |
| MOD (Mininum Object Distance) | 0,8 m |
| llumination | Infrared |
| Housing | Outdoor |
| Color (housing) | black, Pantone P 179-1, RAL9003 |
| Hosuing material | Plastic |
| Heater | no |
| Blower | no |
| Rotation range | 355° |
| Speed (manual) | 45°/Sek. |
| Speed (preset) | 45°/Sek. |
| Alarm inputs | nein |
| Alarm outputs | no |
| Audio support | yes |
| Ethernet interface | 100Base-TX, 10Base-T |
| Serial interfaces | Ethernet |
| Internal storage | microSD-Kartenslot |
| Analysis functions | Attribute Detection (AD), Cross Counting (CC), Crowd Density Detection (CD), Face Attendance (FA), Face Detection (FD), Face Recogntion (FR), Heat Map (HM), Human&Vehicle Detection (PD&VD), License Plate Detection (LPD), License Plate Recognition (LPR), Line Crossing Detection (LCD), Perimeter Intrusion Detection (PID), Queue Lenght Detection (QD), Rare Sound Detection (RSD), Stationary Object Detection (SOD) |
| Mounting | Ceiling mount, Wall mount |
| Supply voltage | 12VDC, PoE |
| Power consumption | 13W |
| Vandal proof | no |
| Operating temperature | -30°C ~ +60°C, @ Humidity 90% or less (non-condensing) |
| ONVIF | ONVIF Profile G, ONVIF Profile S, ONVIF Profile T |
| Protection rating | IP66 |
| Low Speed Shutter | yes |
| Backlight Compensation | BLC, HLC |
| Wide Dynamic Range (WDR) | D-WDR |
| Motion Detector | yes |
| Camera type | PTZ |
| Audio | 103±3dB max., 2 Way Audio, built-in loudspeaker, built-in microphone |
| Remote configuration | Browser |
| Illumination range | approx. 30m |
| Weight | 920g |
| Manufacturer order number | 234752 |

Additional Productinformation

<p>Simple PTZ dome camera for remote pan/tilt, zoom and focus adjustment. </p><p>Not suitable for continuous operation.</p><p><br></p><p style='margin:0cm;font-size:15px;font-family:"Calibri",sans-serif;'><br></p>

Related products

| eneo INsight Installation Tool | Supports adjustments for cameras with AI features for IN-series |
| --- | --- |
| eneo INsight mobile | Mobile App for recorder IN- and MN-series |
| eneo Site Manager | Management Software for eneo IP products |
| KBD-3NET | Network keyboard with joystick, supports ONVIF protocol, RJ45, 12VDC |

Recommendations

| eneo INsight mobile | Mobile App for recorder IN- and MN-series |
| --- | --- |
| jetpower-PSU-S | Universal Netzteil 12V/5A ID 2.1 x OD 5.5 |

Services

| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |
| --- | --- |