Specifications text

Article number

234752

Type

eneo INP-58M2812M0A

Short description

Network Setup-PTZ Dome Camera, 4x MVF, 3840x2160, Day/Night, DWDR, AI, 2,8-12mm, Audio, IR, 12VDC, PoE, IP66

Main Features

1/2.7" CMOS sensor, maximum resolution 8 MP/30fps

AI analysis functions including object classification

D-WDR, 3D-DNR noise reduction

Motorized varifocal lens 2.8-12mm

Smart Codec H.265/H.264, multi-streaming

Browser independent (IE, Edge, Safari, Firefox, Chrome)

Supports ONVIF profiles S/T/G/M

Local storage via Micro SD card

Supports audio

IR LED lighting, IP66, deterrent (light & siren)

NDAA compliance

Wall and ceiling mounting included

Other Features (Specifications)

| General |  |
| --- | --- |
| Heating | No |
| Integrated wall mount | Yes |
| Audio compression | G.711 |
| Lighting | Infrared |
| Dimensions (WxHxD) (extended) | 166,75x238mm |
| Illumination range | Approx. 30m |
| ESC | 1/5 ~ 1/20.000sec. |
| Weight | 0.92 kg |
| Text overlay | Switchable on/off |
| Color (Housing) | RAL9003,Pantone P 179-1,black |
| Housing type | Outside |
| Housing material | Plastic |
| Camera Type | Domecamera |
| Power consumption | 13 W |
| Mounting type | Ceiling, Wall |
| ONVIF | ONVIF Profile G, ONVIF Profile M, ONVIF Profile S, ONVIF Profile T |
| Power over Ethernet | IEEE 802.3af |
| Protection | IP66 |
| Power supply | 12VDC, Poe |
| Temperature Range (Operating) | -30°C ~ +60°C,@ Luftfeuchtigkeit 90% oder weniger (nicht kondensierend) |
| Wide Dynamic Range (WDR) | D-WDR |
| Camera |  |
| Backlight compensation | Yes |
| Low Speed Shutter | Yes |
| Resolution standard | 8 MP |
| Recording sensor | Chipset NT98528, CMOS, SC8238 |
| Chip size | 1/2,7" |
| Flickerless function | Yes |
| Camera shape | Minsters |
| Manual Electric Shutter (MES) | 1-5~1-20,000sec. |
| Day/night mode | Automatic,Day,Night,Image,Time |
| White balance | Automatic,Manual |
| Photosensitivity | 0,009 Lux bei F1,6 |
| System | True Day&Night |
| Day/night switching | Switchable IR Filter (ICR) |
| Display |  |
| Image resolution max. | 3840x2160 |
| DORI |  |
| Watch (62px/m) | 22,4 - 111,5 m |
| Detect (125px/m) | 10,6 - 55 m |
| Identify (250px/m) | 5 - 27,3 m |
| Perceiving (25px/m) | 57,3 - 277,5 m |
| Functions |  |
| Analysis | Abandoned and Removed Objects (SOD), Attendance (FA), Crowds (CD), Enter Region, Face Attributes (AD), Face Capture (FD), Face Recognition (FR), Heat Map (HM), Intrusion, Intrusion (PID), Leave Region, License plate recognition (LPD), License plate recognition (LPR), Line crossing (LCD), Manipulation (TD), Object Classification Human / Vehicle (PD&VD), Passage count (CC), Queues (QD), Rare Sounds (RSD) |
| Audio support | Yes |
| Motion detector | Yes |
| Digital Noise Reduction (DNR) | Yes |
| Multicast | Yes |
| Turning range | 355 ° |
| Alarm processing | recording internal storage (SD card),E-Mail,HTTP,FTP,TCP,UDP,Red & bule flashing lights,Siren,Audio warning message,Cloud Upload |
| Audio | 103±3dB max., 2 Way Audio, Built-in microphone, Built-in speaker |
| Browser | Edge, Firefox, Google Chrome, Safari |
| Home Position | no |
| Menu languages | English, French, German, Italian, Japanese, Portuguese, Russian, Spanish, Turkish |
| Password | Administrator and User |
| Privacy Zone | Areas-ON/OFF switchable |
| Tours | no |
| Speed (manual control) | 45 °/sec |
| Speed (Target Approach) | 45 °/sec |
| Internal memory | microSD card slot |
| Notes |  |
| Remark | Face and license plate recognition require an SD card or an eneo IN recorder. For optimal results,use only eneo components. For mixed brands,we offer a compatibility check for a fee (info: item #229235 at VIDEOR). |
| Interfaces / Inputs/ Outputs |  |
| External connections | RJ-45,12VDC input (DC plug),connection cable approx.45cm,micro SD card slot 256 GB |
| Ethernet interface | 100Base-TX, 10Base-T |
| Control Interfaces | Ethernet |
| Parts supplied |  |
| Scope of delivery | Quick installation guide,drilling template,4 wall plugs with screws,1 Allen wrench,waterproof cover for RJ45 connector |
| Network |  |
| Network Protocols | DDNS, DHCP, DNA, FTP, HTTP, HTTPS, IEE 802.1, NTP, RTP, SMTP, TCP-IP, UPnP |
| Lens |  |
| Horizontal angle of view | 113 - 31,4° |
| Vertical angle of view | 58,4 - 17,6 |
| Focal length | 2,8 - 12 mm |
| Aperture Range (F) | F1.6 |
| MOD (Minimum Object Distance) | 0.8 m |
| Lens Type | Varifocal (motorized) |
| Zoomfactor (optical) | 4 |
| Video |  |
| Image transfer rate max. | 25 fps |
| Resolution (extended) | 3840x2160 (16:9),2592x1944 (4:3),2592x1520 (16:9),2304x1296 (16:9),1920x1080 (16:9),1280x960 (4:3),1280x720 (16:9),640x480 (4:3),320x240 (4:3) |
| Picture size | 16:9, 4:3 |
| Data stream types | Triple-Stream(Main/Sub/Mobile) |
| Video Compression | H.264, H.264+, H.265, H.265+, M-JPEG |
| Product labeling: |  |
| EAN/GTIN | 4260229840115 |
| Manufacturer order number | 234752 |
| Series | eneo IN-Serie |
| Brand | eneo |

Additional Productinformation

Related products

| eneo INsight | Control software for recorder IN- and MN-series |
| --- | --- |
| eneo INsight Installation Tool | Supports adjustments for cameras with AI features for IN-series |
| eneo INsight mobile | Mobile App for recorder IN- and MN-series |
| eneo KBD-3NET | Network keyboard with joystick, supports ONVIF protocol, RJ45, 12VDC |
| eneo PM-21 | Pole mounting adapter, steel for camera connection box AK-20, AK-21 and wall mount WB-24 |
| eneo PM-21 | Pole mounting adapter V2A stainless steel, for camera connection box AK-20, AK-21 and wall mount WB-24 |
| eneo Site Manager | Management Software for eneo IP products |

Recommendations

| eneo NSMI-4004M0A | Gigabit Switch Industrial, Managed, 4x PoE, 2xSFP, 30W each Port, -40°C~75°C, 44~57 VDC, w/o power supply |
| --- | --- |
| eneo NSMI-4008M0A | Gigabit Switch Industrial, Managed, 8x PoE, 2xSFP, 30W each Port, -40°C~75°C, 44~57 VDC, w/o power supply |
| eneo NSMR-2008M0A | Gigabit Switch Rack, Managed, 8x PoE, 2xSFP, max. 30W each Port, internal Power Supply max. 150W |
| eneo NSMR-2016M0A | Gigabit Switch Rack, Managed, 16x PoE, 2xSFP, max. 30W each Port, integrated Power Suppply max. 250W |
| eneo NSMR-2024M0A | Gigabit Switch Rack, Managed, 24x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 400W |
| eneo NSUI-3004M0A | Gigabit Switch Industrial, Unmanaged, 4x PoE, 1xRJ45, 1xSFP, 30W each Port, -40°C~75°C, 44~57 VDC, w/o power supply |
| eneo NSUI-3008M0A | Gigabit Switch Industrial, Unmanaged, 8x PoE, 2xSFP, 30W each Port, -40°C~75°C, 44~57 VDC, w/o power supply |
| eneo NSUR-1008M0A | Gigabit Switch Rack, Unmanaged, 8x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 150W |
| eneo NSUR-1016M0A | Gigabit Switch Rack, Unmanaged, 16x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 250W |
| eneo NSUR-1024M0A | Gigabit Switch Rack, Unmanaged, 24x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 400W |
| Jetrics jetcat6SFTP-GN-0,5/1 |  |
| Jetrics jetcat6SFTP-GN-1/1 |  |
| Jetrics jetcat6SFTP-GN-2/1 |  |
| Jetrics jetcat6SFTP-GN-3/1 |  |
| Jetrics jetcat6SFTP-GN-7,5/1 |  |
| Jetrics jetpower-PSU-S | Universal PSU 12V/5A ID 2.1 x OD 5.5 |

Services

| Vorkonfiguration IP-Kamera | Preconfiguration 1x IP camera Function test, FW update |
| --- | --- |