Specifications text

Article number

233904

Type

eneo INR-18N040005A

Short description

Main Features

Maximum resolution 8MP (recording)

Recording and research AI analysis functions

Recording modes continuous, manual or AI-based

Maximum data rate input 50Mbps, output 64Mbps

Various options for alarm messages

4 x RJ45 PoE port (45 Watt PoE budget total)

Maximum 1 internal hard disk possible

Supports 4 x alarm inputs and 1 x alarm output

2 x USB inputs, 1 x RS-485, 1 x audio input and output

eSATA port for external storage expansion

NetDrive or cloud storage (Google Drive/Dropbox)

Remote access via browser, eneo Insight or Insight mobile

Other Features (Specifications)

| System requirements |  |
| --- | --- |
| Operating system | Linux |
| General |  |
| Dimensions (WxHxD) (extended) | 300x53x227mm |
| External Eneo Operating Devices | Mouse |
| Control software | eneo INsight |
| HDD Slots Number | 1,bis zu 10 TB je slot (Enterprise-Class) |
| Power consumption | 5 W |
| ONVIF | ONVIF Profile S |
| Power supply | 48VDC |
| Temperature Range (Operating) | 0°C ~ +40°C |
| Recording |  |
| Resolution Recording (max) | 3840x2160 |
| Hard disk capacity max | 10 TB |
| Functions |  |
| Audio support | Yes |
| HDD Hotswap | No |
| Analysis | Depending on connected IP camera |
| Alarm triggers | AI, Contact, Disk Error, Image loss, Movement |
| Alarm response | Alarm Output, Buzzer, E-mail dispatch, FTP, Live Popup, PTZ Preset, Push (Mobile), Spot Popup |
| Menu languages | Arabic, Bulgarian, Danish, Dutch, English, Finnish, French, German, Greek, Hebrew, Hindi, Italian, Japanese, Korean, Polish, Portuguese, Romanian, Russian, Spanish, Thai, Turkish |
| Password | Administrator and User |
| Search function | Event, Instant playback, Museum, Text, Thumbnail, VCA Search |
| Remote Access | Android, Ios, Remote Software, WEB Browser (Java) |
| Notes |  |
| Remark | IP cameras of the PXB,PXC,PXD and PTB series are not supported by this product. NDAA compliance under Section 889 |
| Interfaces / Inputs/ Outputs |  |
| Alarm Input | Yes |
| Alarm output | Yes |
| Number of IP Channels | 4 |
| Audio Inputs | 1 |
| Ethernet interface | 100Base-TX, 10Base-T |
| Memory interface (external) | USB |
| Control Interfaces | RS-485 |
| Video Outputs | 1x HDMI, VGA |
| Video Inputs (Quantity) | 4 |
| Parts supplied |  |
| Scope of delivery | PSU,Power cord,HDD assembly-kit,mouse,manual |
| Network |  |
| Data throughput (outbound) | 64 Mbps |
| Data throughput (maximum) | 64 Mbps |
| Network Protocols | DDNS, DHCP, DNA, FTP, HTTP, HTTPS, RTP, RTSP, SMTP, TCP-IP, UPnP |
| Video |  |
| Video Compression | H.264, H.264+, H.265, H.265+ |
| Product labeling: |  |
| EAN/GTIN | 4260229840108 |
| Manufacturer order number | 233904 |
| Series | eneo IN-Serie |
| Brand | eneo |

Related products

| eneo INR-eSATA |  |
| --- | --- |
| eneo INsight | Control software for recorder IN- and MN-series |
| eneo INsight Evidence Locker | For the research of HDD's for recorders of IN-series |
| eneo INsight Evidence Player | For replay of exported data for recorder IN-series |
| eneo INsight mobile | Mobile App for recorder IN- and MN-series |
| eneo Site Manager | Management Software for eneo IP products |
| Western Digital WD44PURZ | WD Purple Hard Drive, 3.5", SATA 6 Gb/s 4TB, 128MB Cache, 24x7, optimized for video surveillance |
| Western Digital WD64PURZ | WD Purple hard drive, 3.5", SATA 6 Gb/s, 6TB, 256MB Cache, 24x7, optimized for video surveillance |
| Western Digital WD85PURZ |  |

Recommendations

| eneo NSMI-4004M0A | Gigabit Switch Industrial, Managed, 4x PoE, 2xSFP, 30W each Port, -40°C~75°C, 44~57 VDC, w/o power supply |
| --- | --- |
| eneo NSMI-4008M0A | Gigabit Switch Industrial, Managed, 8x PoE, 2xSFP, 30W each Port, -40°C~75°C, 44~57 VDC, w/o power supply |
| eneo NSMR-2008M0A | Gigabit Switch Rack, Managed, 8x PoE, 2xSFP, max. 30W each Port, internal Power Supply max. 150W |
| eneo NSMR-2016M0A | Gigabit Switch Rack, Managed, 16x PoE, 2xSFP, max. 30W each Port, integrated Power Suppply max. 250W |
| eneo NSMR-2024M0A | Gigabit Switch Rack, Managed, 24x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 400W |
| eneo NSUI-3004M0A | Gigabit Switch Industrial, Unmanaged, 4x PoE, 1xRJ45, 1xSFP, 30W each Port, -40°C~75°C, 44~57 VDC, w/o power supply |
| eneo NSUI-3008M0A | Gigabit Switch Industrial, Unmanaged, 8x PoE, 2xSFP, 30W each Port, -40°C~75°C, 44~57 VDC, w/o power supply |
| eneo NSUR-1008M0A | Gigabit Switch Rack, Unmanaged, 8x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 150W |
| eneo NSUR-1016M0A | Gigabit Switch Rack, Unmanaged, 16x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 250W |
| eneo NSUR-1024M0A | Gigabit Switch Rack, Unmanaged, 24x PoE, 2xSFP, max. 30W each Port, integrated Power Supply max. 400W |
| Jetrics jetcat6SFTP-GN-0,5/1 |  |
| Jetrics jetcat6SFTP-GN-1/1 |  |
| Jetrics jetcat6SFTP-GN-2/1 |  |
| Jetrics jetcat6SFTP-GN-3/1 |  |
| Jetrics jetcat6SFTP-GN-7,5/1 |  |
| Jetrics jetcon HDMI Kabel 1 |  |
| Jetrics jetcon HDMI Kabel 2 |  |
| Jetrics jetcon HDMI Kabel 3 |  |

Services

| CON-eneo-KON-04 | Preconfiguration 1x eneo recorder with up to 4 eneo IP cameras |
| --- | --- |
| CON-HDD-eneo | Hard disk installation service for eneo |
| CON-KOMP | Compatibility check for cameras, NVR, VMS systems (one pair of devices) |