Specifications text

Article number

234750

Type

INR-28N320005A

Short description

Network Video Recorder 32x IP total, 16x IP PoE, HDMI, 4xSATA, w/o HDD

Main Features

Maximum resolution 8MP (recording)

Recording and research AI analysis functions

Recording modes continuous, manual or AI-based

Maximum data rate input 256Mbps, output 128Mbps

Various options for alarm messages

Maximum 4 internal hard disk possible

Supports 16 x alarm inputs and 1 x alarm output

3 x USB inputs, 1 x RS-485, 1 x audio input and output

eSATA port for external storage expansion

NetDrive or cloud storage (Google Drive/Dropbox)

Remote access via browser, eneo Insight or Insight mobile

32 IP channels, 16 x RJ45 PoE port (220 Watt PoE budget)

Other Features (Specifications)

| Brand | eneo |
| --- | --- |
| EAN/GTIN | 4260229840016 |
| Dimensions (HxWxD) | 378x325,8x66mm |
| Series | eneo IN-Series |
| Blower | yes |
| Channels total | 32 |
| IP channels | 32 |
| Max. recording resolution | 3840x2160 |
| Compression | H.264, H.264+, H.265, H.265+ |
| Spot monitor | no |
| Monitor Outputs | 1x HDMI (2MP), 1x HDMI (4K), VGA |
| Network | 1x 10/100/1000 Base-T RJ45 |
| Alarm inputs | 16 |
| Alarm output | 1 |
| Audio support | yes |
| Audio inputs | 1 |
| Serial interfaces | RS-485 |
| Max. internal HDD capacity | 40 TB |
| Internal HDD capacity | 0 TB |
| Integrated writer | no |
| Interfaces external storage | USB |
| HDD slots | 4x SATA |
| HDD hotswap | no |
| USB ports | 1x USB 3.0, 2x USB 2.0 |
| RAID | n/a |
| Data tranfer rate max. | 384 Mbps |
| Analysis functions | Depending on connected IP camera |
| Remote access | Android, iPhone, remote software, WEB Browser (Java) |
| 19" rack mount | yes |
| Supply voltage | 230VAC |
| Power consumption | 12W, without HDD |
| Power over Ethernet | IEEE 802.3af, IEEE 802.3at |
| PoE budget | 220 W |
| Operating temperature | 0°C ~ +40°C |
| ONVIF | ONVIF Profile S, yes |
| Data transfer rate outgoing | 128 Mbps |
| Remote configuration | Remote software |
| Weight | 2.95kg |
| Playback | 16 channel split, 4 channel split, 8 channel split, full screen, sequence |
| Network connection | 1 x 10/100/1000BASE-T, 16x PoE RJ45 10/100 Base-T |
| Data transfer rate incoming | 256 Mbps |
| Audio output | 1 |
| Operating system | Linux |
| Manufacturer order number | 234750 |

Additional Productinformation

Related products

| eneo INsight | Control software for recorder IN- and MN-series |
| --- | --- |
| eneo INsight Evidence Locker | For the research of HDD's for recorders of IN-series |
| eneo INsight Evidence Player | For replay of exported data for recorder IN-series IN-Serie |
| eneo INsight mobile | Mobile App for recorder IN- and MN-series |
| HDD-2000SATA Purple | Hard Disk Drive 2TB, SATA, for eneo Video Recorders, expansion |
| HDD-3000SATA Purple | Hard Disk Drive 3TB, SATA, for eneo Video Recorders, expansion |
| HDD-4000SATA Purple | Hard Disk Drive 4TB, SATA, for eneo Video Recorders, expansion |
| HDD-6000SATA Purple | Hard Disk Drive 6TB, SATA, for eneo Video Recorders, expansion |
| INR-eSATA | HDD Box, eSATA, für max 4 HDD max. 8TB pro HDD, ohne HDD |

Recommendations

| DS223 | Network Attached Storage, 2-Bay, Hotswap, ohne HDD, Quad core, 2GB RAM |
| --- | --- |
| eneo INsight mobile | Mobile App for recorder IN- and MN-series |

Services

| CON-eneo-KON-16 | Preconfiguration 1x eneo recorder with up to 16 eneo IP cameras |
| --- | --- |
| CON-eneo-KON-32 | Preconfiguration 1x eneo recorder with up to 32 eneo IP cameras |
| CON-HDD-eneo | Hard disk installation service for eneo |
| CON-KOMP | Compatibility check for cameras, NVR, VMS systems (one pair of devices) |