Specifications text

Article number

234749

Type

INR-28N080005A

Short description

Network Video Rekorder 8x IP PoE, H.265, HDMI, VGA 1xSATA, w/o HDD

Main Features

Maximum resolution 8MP (recording)

Recording and research AI analysis functions

Recording modes continuous, manual or AI-based

Maximum data rate input 128Mbps, output 128Mbps

Various options for alarm messages

8 x RJ45 PoE port (59 Watt PoE budget total)

Maximum 1 internal hard disk possible

Supports 8 x alarm inputs and 1 x alarm output

2 x USB inputs, 1 x RS-485, 1 x audio input and output

eSATA port for external storage expansion

NetDrive or cloud storage (Google Drive/Dropbox)

Remote access via browser, eneo Insight or Insight mobile

Other Features (Specifications)

| EAN/GTIN | 4260229840085 |
| --- | --- |
| Alarm output | Yes |
| Alarm Input | No |
| Analysis | Depending on connected IP camera |
| Audio Inputs | 1 |
| Audio support | Yes |
| Resolution Recording (max) | 3840x2160 |
| Data throughput (maximum) | 256 Mbps |
| Alarm triggers | AI, Contact, Disk Error, Image loss, Movement |
| Alarm response | Alarm Output, Buzzer, E-mail dispatch, FTP, Live Popup, PTZ Preset, Push (Mobile), Spot Popup |
| External Eneo Operating Devices | Mouse |
| Remark | "IP cameras of the PXB,PXC,PXD and PTB series are not supported by this product,NDAA compliance under Section 889,Note: For two freely definable channels,the alarm recording can be saved in the cloud.,To guarantee you an optimum user experience,we recomme |
| Scope of delivery | PSU,HDD assembly-kit,mouse,manual |
| Menu languages | Arabic, Bulgarian, Danish, Dutch, English, Finnish, French, German, Greek, Hebrew, Hindi, Italian, Japanese, Korean, Polish, Portuguese, Romanian, Russian, Spanish, Thai, Turkish |
| Password | Administrator and User |
| Control software | eneo INsight |
| Search function | Event, Instant playback, Museum, Text, Thumbnail, VCA Search |
| HDD Hotswap | No |
| Manufacturer order number | 234749 |
| RAID Level | no |
| Remote Access | Android, Ios, Remote Software, WEB Browser (Java) |
| Memory interface (external) | USB |
| Series | eneo IN-Serie |
| Control Interfaces | RS-485 |
| Temperature Range (Operating) | 0°C ~ +40°C |
| Brand | eneo |

Additional Productinformation

Related products

| eneo INsight | Control software for recorder IN- and MN-series |
| --- | --- |
| eneo INsight Evidence Locker | For the research of HDD's for recorders of IN-series |
| eneo INsight Evidence Player | For replay of exported data for recorder IN-series |
| eneo INsight mobile | Mobile App for recorder IN- and MN-series |
| HDD-2000SATA Purple | Hard Disk Drive 2TB, SATA, for eneo Video Recorders, expansion |
| HDD-3000SATA Purple | Hard Disk Drive 3TB, SATA, for eneo Video Recorders, expansion |
| HDD-4000SATA Purple | Hard Disk Drive 4TB, SATA, for eneo Video Recorders, expansion |
| HDD-6000SATA Purple | Hard Disk Drive 6TB, SATA, for eneo Video Recorders, expansion |
| INR-eSATA |  |

Recommendations

| DS223 | Network Attached Storage, 2-Bay, Hotswap, ohne HDD, Quad core, 2GB RAM |
| --- | --- |
| eneo INsight mobile | Mobile App for recorder IN- and MN-series |

Services

| CON-HDD-eneo | Hard disk installation service for eneo |
| --- | --- |
| CON-KOMP | Compatibility check for cameras, NVR, VMS systems (one pair of devices) |
| CON-NVR-KON-08 | Preconfiguration 1x Network Video Recorder with up to 8 IP cameras |