Specifications text

Article number

233903

Type

MNR-18N04000A

Short description

Hybrid HD Video Recorder 4-channel analogue, 2-ch. IP, HDMI, 1xSATA, w/o HDD

Main Features

4-channel pentaplex multisignal recorder

Supports IP, HD-TVI, AHD, CVBS signals

Max. 6 channels (4x analog + 2x IP)

Recording speed 28fps@8MP,48fps@5MP,120fps@2MP

Monitor outputs HDMI (4K),VGA and spot monitor CVBS

Maximum 1 internal hard disk possible

PTZ control via coaxial (UTC)

Audio/alarm inputs and outputs

USB 2.0 and RS-485 interface

Compression methods: H.264, H.265

Remote access via app, browser and CMS

USB mouse included

Other Features (Specifications)

| EAN/GTIN | 4260229840252 |
| --- | --- |
| Alarm inputs | 4 |
| Audio Inputs | 4 |
| Resolution Recording (max) | 3840x2160 |
| Recording Rate Maximum | 120 fps |
| Alarm triggers | AI, Contact, Disk Error, Image loss, Movement |
| Alarm response | Alarm Output, Buzzer, E-mail dispatch, FTP, Live Popup, PTZ Preset, Push (Mobile), Spot Popup |
| External Eneo Operating Devices | Mouse |
| Remark | "NDAA compliance under Section 889,Note: For two freely definable channels,the alarm recording can be saved in the cloud.,To guarantee you an optimum user experience,we recommend that you only use components of the same brand in a system. If several brand |
| Scope of delivery | PSU,HDD assembly-kit,mouse,manual |
| Menu languages | Arabic, Bulgarian, Danish, Dutch, English, Finnish, French, German, Greek, Hebrew, Hindi, Italian, Japanese, Korean, Polish, Portuguese, Romanian, Russian, Spanish, Thai, Turkish |
| Password | Administrator and User |
| Search function | Event, Instant playback, Text, Thumbnail |
| Hard drive capacity (internal) | 0 TB |
| Hard drive capacity (max internal) | 10 TB |
| IP Channels | 2 (6 IP only) |
| Total Channels | 6 |
| Drive (internal) | no |
| Manufacturer order number | 233903 |
| Remote Access | Microsoft Internet Explorer, Remote Software |
| Memory interface (external) | USB |
| Series | eneo IN-Serie |
| Spot Monitor | Composite (FBAS) |
| Control Interfaces | Coaxitron (HD Protocol), RS-485 |
| Video Outputs | HDMI (4K), VGA |
| Video Inputs (Quantity) | 4 |
| Brand | eneo |

Additional Productinformation

Related products

| eneo INsight | Control software for recorder IN- and MN-series |
| --- | --- |
| eneo INsight Evidence Locker | For the research of HDD's for recorders of IN-series |
| eneo INsight Evidence Player | For replay of exported data for recorder IN-series |
| eneo INsight mobile | Mobile App for recorder IN- and MN-series |
| HDD-2000SATA Purple | Hard Disk Drive 2TB, SATA, for eneo Video Recorders, expansion |
| HDD-3000SATA Purple | Hard Disk Drive 3TB, SATA, for eneo Video Recorders, expansion |
| HDD-4000SATA Purple | Hard Disk Drive 4TB, SATA, for eneo Video Recorders, expansion |
| HDD-6000SATA Purple | Hard Disk Drive 6TB, SATA, for eneo Video Recorders, expansion |

Recommendations

| eneo INsight mobile | Mobile App for recorder IN- and MN-series |
| --- | --- |

Services

| CON-eneo-KON-04 | Preconfiguration 1x eneo recorder with up to 4 eneo IP cameras |
| --- | --- |
| CON-HDD-eneo | Hard disk installation service for eneo |
| CON-KOMP | Compatibility check for cameras, NVR, VMS systems (one pair of devices) |